Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	POWER SYSTEM ANALYSIS							      Time : 3 hours

Subject Code:	EE255											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define the transient state of power system.

2.	What are the merits of per unit representation?

3.	What are the different types of power system buses?

4.	Write the merits of Newton Raphson method.

5.	What is the use of accelerating factor?

6.	Write the significance of symmetrical components.

7.	Write the importance of short circuit studies.

8.	What is meant by unit commitment?

9.	How do you classify the stability studies?

10.	Write the concept of critical clearing angle.

PART – B  (5 x 3 = 15 MARKS)

11.	How do you fix the real and reactive power limits for the generator?

12.	What are the types of contingencies?

13.	What are the different types of power system faults?

14.	Write the concept of generator incremental cost.

15.	Write the merits of Runge-Kutta method.

PART – C  (5 x 15 = 75 MARKS)

16.	Briefly explain the modeling of synchronous generator, transformer, transmission line and loads of the power system.

(OR)

17.	Explain the procedure to compute the symmetrical components of the power network when it is subjected to fault conditions.

18.	Describe the step by step algorithm for bus impedance matrix of a sample system with and without mutual coupling.

(OR)

19.	Derive the fault current expression for line to line fault occurs in a real power system and explain its significance.

20.	Obtain the load flow solution of the power system using Gauss-Seidel method with flow chart. (OR)

21.	Explain the load flow solution procedure of fast decoupled method and compare its performance with Newton-Raphson method.

22.	From the fundamentals, derive the loss formula for the transmission system.

(OR)

23.	Explain the procedure to determine optimum scheduling condition between hydro-thermal power plants in detail.

[P.T.O]

24.	Obtain the stability condition of the fault constrained system using Equal area criteria. Draw the necessary illustrations.

(OR)

25.	Explain the performance of multimachine stability analysis using Runge-Kutta method.