Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	LINEAR INTEGRATED CIRCUITS						      Time : 3 hours

Subject Code:	EE258											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	An ideal Op-Amp has infinite input impedance. True/False.

2.	Open loop gain of practical Op-Amp decreases with increasing frequency. True/False.

3.	An integrator is basically a ___________(low/high) pass filter.

4.	Differentiators are not preferred in analog computers as they amplify noise. True/False.

5.	An Op-Amp without feedback may act as a comparator. True/False.

6.	The cutoff voltage of Precision rectifier is exactly 0.707V. True/False.

7.	The value of Load regulation of a good regulator should be minimum. True/False.

8.	In a PLL, Lock-in range is always less than Capture range. True/False.

9.	There is a relationship between number of bits and resolution in DAC. True/False.

10.	___________ ADC has excellent noise rejection.

PART – B  (5 x 3 = 15 MARKS)

11.	Define Slew Rate.

12.	Draw the circuit for Voltage follower using Op-Amp.

13.	Draw the circuit of zero crossing detector.

14.	What are three terminal regulators?

15.	Define resolution of a DAC.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail ideal characteristics of Op-Amp.

(OR)

17.	Mention and explain different AC characteristics of an Op-Amp.

18.	Draw the circuit of RC phase shift oscillator and explain its operation.

(OR)

19.	Design a third order high pass active filter with cutoff frequency of 10 kHz.

20.	Draw the circuit of regenerative comparator  and explain its operation.

(OR)

21.	Draw the circuit of precision full wave rectifier and explain.

22.	Draw the Astable multi using IC 555 and explain its operation.

(OR)

23.	With neat sketch explain in applications of PLL in communication.

24.	Draw the diagram of weighted resistor DAC and explain its operation.

(OR)

25.	With neat sketch explain the operation of Successive approximation ADC.