Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MEASUREMENTS AND INSTRUMENTATION			      Time : 3 hours

Subject Code:	EE262							      		       		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The power consumption in PMMC instruments is typically about ___________.

2.	Which meter has highest accuracy in the prescribed limit of frequency range?

3.	In electrodynamometer type of wattmeter, the current coils use stranded wire or laminated conductors to reduce _____________.

4.	Define the term Calibration.

5.	Name the bridge that is used for measurement of very low values of resistance.

6.	Maxwell’s inductance-capacitance bridge is used for measurement of inductance of ___________.

7.	LVDT stands for _____________.

8.	Name the transducer used for measurement of temperature.

9.	Mention the different types of voltmeters.

10.	List the functions of signal generators.

PART – B  (5 x 3 = 15 MARKS)

11.	How a PMMC meter can be used as voltmeter and ammeter?

12.	Why is an electrodynamometer a transfer instrument?

13.	Write the balance equation of a Wheatstone bridge.

14.	What is the principle of a piezo electric transducer?

15.	What is the function of a oscilloscope?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about the principle, construction and working of D’ Arsonval galvanometer with a neat diagram.

(OR)

17. 	Explain in detail about the principle, construction and operation of moving coil and moving iron instruments with a neat sketch.

18.	Describe the constructional details of an electrodynamometer type wattmeter with a neat diagram.

(OR)

19.	 Explain the construction and theory of operation of a single phase induction type energy meter.

20.	Draw and explain the circuit of a Wheatstone bridge. Explain how the effects of precision measurement of medium resistance are minimized.

(OR)

21. 	Describe the working of Hay’s bridge for the measurement of inductance. Derive the equations for balance and draw the phasor diagram under condition of balance.

22. 	Explain in detail about the construction, operation and characteristics of LVDT.

[P.T.O.]

(OR)

23.	 Explain in detail about the construction and working principle of RTD with a neat sketch.

24. 	Explain any one type of digital voltmeter with a neat sketch and mention its merits and demerits.

(OR)

25. 	Discuss in detail the principle and working of X-Y recorder with a neat sketch.