Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	MICROPROCESSORS AND MICROCONTROLLERS    	      Time : 3 hours

Subject Code:	EE263										    	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	8085 Microprocessor is a ___________ bit Processor.

2.	What is the role of program counter?

3.	Buffers are used for ___________.

4.	Port A of 8255 is used as___________ in mode 2 operation.

5.	Name the non-maskable interrupt of 8085.

6.	In DMA controller, EOP is used for ___________.

7.	Write the bit pattern of 7 segment display code.

8.	In ADC 0801, the end of conversion will be indicated as ___________ to the processor.

9.	What is the use of data pointer in 8051 microcontroller?

10.	How many I/O Ports are available in 8051 microcontroller?

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the instruction cycle for MVI A, 08H.

12.	Write the control word format in 8255.

13.	Mention the different signals used for communication between microprocessor and DMA controller.

14.	Write a program for frequency measurement using 8085.

15.	Name the different addressing modes of 8051 microcontroller.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw and explain the architecture of 8085 processor.

(OR)

17.	a. 	Briefly write about the addressing modes of 8085 with suitable example.			(7)

b. 	Draw the timing diagram of IN 05H. 											(8)

18.	Discuss the operation of 8255 PPI with suitable diagram.

(OR)

19.	Explain the interfacing of I/O devices using memory mapped I/O technique with diagrams.

20.	a. 	Discuss about the different types of interrupts with example. 						(5)

b. 	With suitable diagram explain the multiple interrupt handling methodology.			(10)

(OR)

21.	What is the need of DMA controller? Explain the working of DMA Controller with functional block diagram.

22.	Explain the multiplexed seven segment LED display system interfacing with 8085 using 8255 PPI.

(OR)

23.	Briefly write about the stepper motor control using 8085 processor.

[P.T.O.]

24.	Draw and explain the architecture of 8051 microcontroller.

(OR)

25.	a. 	Write a program to multiply two 8 bit numbers using 8051. 						(7)

b. 	Discuss the different modes of timers available in 8051. 							(8)