Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	AUTOMOTIVE ELECTRONICS						      Time : 3 hours

Subject Code:	EE294											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Give a list of the various types of sensors used in automotives.

2.	Briefly explain the working of a typical relay.

3.	What is the function of starter switches?

4.	Write down the behavior of starter during starting.

5.	Differentiate Throttle body Injection and Multi port fuel injection system.

6.	List the different types of ignition systems used in vehicles.

7.	What is trafficator?

8.	Speedometer is used to measure ________ in vehicles.

9.	Differentiate open loop and closed loop control systems.

10.	What is acceleration enrichment in digital control systems?

PART – B  (5 x 3 = 15 MARKS)

11.	Why crank angle position and cam position is to be known?

12.	What is the principle of working of  a starter motor?

13.	List the advantages of Electronic ignition systems.

14.	What is insulated and earth return system?

15.	What are the advantages of Electric power steering?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the working of stepper motor with aid of neat sketches.

(OR)

17.	Write short notes on:

a. 	Altitude sensor.				b. 	Throttle position sensors.

18. 	What are the various characteristics of a Series motor and explain its operation.

(OR)

19.  Explain the working of different starter drive units. What is the care and maintenance taken

for  starter during its course of working?

20.	Explain briefly the electronic fuel injection system used in some modern cars.

(OR)

21.	Draw the sketch of a carburetor. How do the idle and low speed circuits work in this   carburetor?

22.	a. 	What do you mean by head light dazzling? How can it be prevented?

b. 	Explain any two types of headlamp with neat diagrams.

(OR)

23.	a. 	With neat sketch explain the working of wiper system of an automobile.

b. 	Explain the working of any two types of horns.

[P.T.O]

24.	Discuss on engine cranking and warm up control in automobiles.

(OR)

25.	Explain the operation of electronically controlled electric brakes.