Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	ELECTRONIC DEVICES AND BASIC CIRCUITS		      Time : 3 hours

Subject Code:	MT224											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Signal is a physical parameter which varies with respect to _________.

2.	Amplifier is the circuit which increases the_________ without changing its _________.

3.	Basic op-amp connections include _________ amplifiers.

4.	For an ideal op-amp the threshold voltage is _________.

5.	The diode current equation is _________.

6.	Other name for JFET source-follower configuration is _________.

7.	Mention one advantage of having voltage divider circuit.

8.	The AC analysis of JFETs and depletion-type MOSFETs is same. True or False?

9.	BJT can act as a switch when it is operated in_________.

10.	BJT can be operated in the following modes such as_________.

PART – B  (5 x 3 = 15 MARKS)

11.	Plot the frequency spectrum of a given signal x (t)= 20 sin400πt+2cos50 πt.

12.	How does the Zener breakdown voltage vary with temperature? What is the breakdown voltage of a diode1Z5.2V?

13.	Specify the characteristics of an ideal op-amp.

14.	What is an IGFET? Write the relation between pinch-off voltage and drain current.

15.	Draw the device structure and input characteristics of BJT.

PART – C  (5 x 15 = 75 MARKS)

16.	Compare analog and digital signal. Draw the circuit models of an amplifier and explain.

(OR)

17.	Illustrate different digital logic inverters.

18. 	a.	Calculate the CMRR (in dB) for the circuit measurements of Vd=1mV, V0=120mVand     	Vc=1mV, Vo=20μV.													    (4)

b.	Determine the output voltage of an op-amp for input voltages of  Vi1=150μV Vi2=140μV. The amplifier has a differential gain of  Ad=4000 and the value of CMMR is : 	i.	100		ii.	105.												    (6)

c.	Draw an  inverting and non-inverting op-amp configuration and express its gain. 	    (5)

(OR)

19.	a. 	Discuss on the effect of finite open-loop gain and bandwidth on circuit performance.	  (10)

b.	Exemplify an integrator circuit.                                                                           		    (5)

20.	a.	With a neat diagram explain the working of a PN junction diode in forward bias and    	reverse bias and show the effect of temperature on its V-I characteristics.                       (8)

b.	Discuss about any one type of special diode and specify its application.                    	    (7)

(OR)

[P.T.O.]

21. 	a.	Differentiate clippers and clampers along with its applications.                                 	    (5)

b.	A full-wave bridge rectifier with a 120-V rms sinusoidal input has a load resistor of                 1KΩ.                                                                                                                            	  (10)

i.	If silicon diodes are employed, what is the DC voltage available at the load?

ii.	Determine the required PIV rating of each diode.

iii.	Find the maximum current through each diode during conduction.

iv.	What is the required power rating of each diode?

22. 	a.	Explain how the transconductance of a JFET varies with drain current and gate voltage   	characteristics and transfer characteristics.                                                     			  (12)

b.	A JFET has the following parameters IDDS = 32 mA, VGS(off) = –8 V and VGS = –4.5V.  Find 	its drain current.                                                                                                     		   (3)

(OR)

23. 	a.	Explain the working of n-channel enhancement type MOSFET. Sketch its typical 	characteristics.                                                                                                           	  (10)

b.	Explain the application of FET as a voltage variable resistor.                              		    (5)

24.  	a.	What is the origin of the name BJT?  Draw the biasing arrangement for CB, CE and CC 	mode.                                                                                                                 		    (8)

b.	With necessary diagrams explain how BJT acts as an amplifier.                              	    (7)

(OR)

25.	a.	Illustrate the frequency response of the CE amplifier.                                             	    (7)

b.	Draw and explain the basic BJT logic inverter.					  		 		    (8)