Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	DIGITAL ELECTRONICS								      Time : 3 hours

Subject Code:	MT227						      		       			       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Convert (634)8 to binary.

2. 	State the full forms of ASCII and EBCDIC code.

3. 	Draw the logic symbol and construct the truth table for the two input EX – OR gate.

4. 	Differentiate encoder and decoder.

5. 	Give the flip-flop excitation tables for T flip-flop.

6. 	How many flip flops are needed to build an 8 bit shift register?

7. 	What are the two models of synchronous sequential circuits?

8. 	What do you mean by next state?

9. 	Draw the CMOS inverter circuit.

10. 	__________ devices consume substantially more power than equivalent CMOS devices.

PART – B  (5 x 3 = 15 MARKS)

11. 	Given the two binary numbers X = 1010100 and Y = 1000011, perform the  subtraction

a.	X Y 	b. Y  X using 2's complements.

12. 	Implement the following Boolean function using 4:1 multiplexer.

F(A,B,C)= Σ m(1,3,5,6)

13. 	Draw the JK Master Slave flip flop circuit.

14. 	Give the comparison between synchronous and asynchronous sequential circuits.

15. 	Explain two input TTL NAND gate.

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Reduce AB + (AC)' + AB'C(AB + C).   											(5)

b.	Convert the given expression in canonical SOP form Y = AC + AB + BC and write the 	maxterms corresponding to the logical expression. 								(10)

(OR)

17.	a. 	Simplify the boolean function by using Quine- McCluskey method.

F(A,B,C,D)= Σ m(0, 2, 3, 6, 7, 8, 10, 12, 13)										(9)

b.	Reduce the following using K-Map method

F(W, X, Y, Z)= Σ m(0, 7, 8, 9, 10, 12) + Σ d(2, 5, 13)                                    			(6)

18.	a. 	Implement a full subtractor with two half- subtractors and an OR gate.  				(8)

b. 	Design a 1:8 demultiplexer using two 1:4  demultiplexers.							(7)

(OR)

19. 	Design a two bit magnitude comparator using logic gates.

20. 	Design and explain the working of a synchronous MOD-3 counter.

(OR)

21.	a. 	Explain a four- bit Johnson counter with timing sequence. 						(10)

b. 	Explain the serial in serial out shift register in detail.  								(5)

[P.T.O.]

22. 	Using SR flip flops, design a parallel counter which counts in the sequence

000,111,101,110,001,010,000 ………

(OR)

23. 	Analyze (State equation, state table, state diagram) the synchronous sequential circuit shown in figure.

24. 	a.	Explain in detail about Schottky TTL logic. 										(7)

b.	Compare RTL, DTl, ECL and TTL.												(8)

(OR)

25. 	Explain the architecture of PLDs and PLA in detail.