Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	FUNDAMENTALS OF ELECTRONIC COMMUNICATION														      								     Time : 3 hours

Subject Code:	MT233											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define noise factor.

2. 	Which circuit is commonly used in video amplifiers?

3. 	Name any one negative resistance oscillator.

4. 	The image rejection in communication receiver can be improved by using ____________.

5. 	For 100% modulation, what percentage of power is in each sideband in AM?

6. 	If the deviation is 75 KHz and maximum modulating frequency is 5 KHz, what is the bandwidth of an FM wave?

7. 	The magnitude of frequency and phase deviation of carrier is proportional to ____________ of modulating signal.

8. 	What is Nyquist sampling rate?

9. 	Mention the distortions present in delta modulation.

10. 	The hamming weight of a codeword 11000100 is ____________.

PART – B  (5 x 3 = 15 MARKS)

11. 	The noise figure of an amplifier is 7dB. Calculate the output signal to noise ratio when the        	input signal to noise ratio is 35dB.

12. 	Explain briefly about frequency synthesizers.

13. 	What are the advantages of FM over AM?

14. 	Write a note on noise in FM systems.

15. 	What are the advantages and disadvantages of digital transmission?

PART – C  (5 x 15 = 75 MARKS)

16. 	What is the need for neutralization? Explain with example circuits, how this is achieved.

(OR)

17. 	Explain the working principle of Class C amplifier and find its efficiency.

18. 	Draw and explain the general tuned LC oscillator circuit. Illustrate how this can be utilized to obtain a Colpitt and Clapp oscillator circuits.

(OR)

19. 	Explain the function of super heterodyne receiver with neat diagram. What are the factors that influence the choice of IF?

20.	a. 	For an AM DSBFC envelope with a +Vmax= 40 V and +Vmin = 10V, determine 	unmodulated  carrier amplitude, modulation index and modulating signal amplitude.   (6)

b.	With neat block diagram, explain the principle of High level AM Transmitter.               (9)

(OR)

21.   Explain in detail about SSB generation methods with relevant diagram.

[P.T.O.]

22. 	a. 	Compare FM and PM.                                                                                                         (5)

b. 	Explain the working principle of any one Angle modulation detector.                             (10)

(OR)

23. 	a. 	Explain the concept of Time Division Multiplexing with a neat sketch.                       	  (10)

b. 	List the advantages and drawbacks of Pulse Time Modulation.                                   	    (5)

24.  	Explain the principle of operation of PCM encoder and decoder with neat block diagrams. What are the advantages of PCM?

(OR)

25. 	Discuss in detail about any two error detection and correction codes with examples.