Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2014

Subject Title : 	OPERATIONS RESEARCH								      Time : 3 hours

Subject Code:	PE237											       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the phases in the study of operation research?

2.	Define a model in operation research.

3.	State any one differences between PERT and CPM?

4.	What is optimum sequence in sequencing problem?

5.	What is EOQ economic order quantity?

6.	In ABC analysis, ABC stands for _______.

7.	What is Transient state in queuing theory?

8.	List the applications of queuing theory.

9.	Define ‘saddle point’?

10.	State the applications of game theory.

PART – B  (5 x 3 = 15 MARKS)

11.	What is degeneracy in transportation problem?

12.	What is ‘NO PASSING RULE’ in sequencing?

13.	Explain the cost involved in inventory problem.

14.	Name the components in the queuing system.

15.	Write a note on group replacement.

PART – C  (5 x 15 = 75 MARKS)

16.	Solve the following LPP [graphically/simplex]

Maximize Z =7x1 + 5x2

STC      	x1 + 2x2  ≤  6,

4x1 + 3x2  ≤ 12,

x1, x2  ≥ 0

(OR)

17.	Find the optimum total transportation cost.

|  | A | B | C | D | E | Supply |
| --- | --- | --- | --- | --- | --- | --- |
| P | 4 | 1 | 2 | 6 | 9 | 100 |
| Q | 6 | 4 | 3 | 5 | 7 | 120 |
| R | 5 | 2 | 6 | 4 | 8 | 120 |
| Demand | 40 | 50 | 70 | 90 | 90 |  |

18.	The project consist of following activities

| Activity | A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1-2 | 1-3 | 1-5 | 2-3 | 2-4 | 3-4 | 3-5 | 3-6 | 4-6 | 5-6 |
| Duration (in weeks) | 8 | 7 | 12 | 4 | 10 | 3 | 5 | 10 | 7 | 4 |

[P.T.O.]

a.	Construct N/W diagram.

b.	Find critical path.

c.	Find minimum time required to complete the project.

(OR)

19.	There are six jobs each of which goes through three machines in the order M1, M2,  M3. Find the sequence which minimizes the total elapsed time and also find idle time of machines in the order M1, M2, M3 (time units in =hrs)

| Jobs | Machine M1 | Machine M2 | Machine M3 |
| --- | --- | --- | --- |
| 1 | 12 | 7 | 3 |
| 2 | 8 | 10 | 4 |
| 3 | 7 | 9 | 2 |
| 4 | 11 | 6 | 5 |
| 5 | 10 | 10 | 5 |
| 6 | 5 | 4 | 4 |

20.	 A company has a demand of 12,000 units/year, and it can produce 2000 such items per month. Carrying cost is `0.15 / month, Cost of one set-up is ` 400, and the shortage cost is

` 20 /year Calculate: - 	a.	The EOQ 	b.	total cost per year (assuming the cost of one unit is ` 4).

(OR)

21.	Explain the ABC analysis in detail with an example.

22.	Arrivals at telephone booth are considered to be Poisson with an average time of   10 minutes between one arrival and the next. The length of phone calls is assumed to be distributed exponentially, with a mean of 3 minutes.

a.	What is the probability that a person arriving at the booth will have to wait?

b.	The telephone department will install a second booth when convinced that an arrival would c.	expect waiting for at least 3 minutes for a phone call. By how much should the flow of 	arrivals increase in order to justify a second booth?

d.	What is the average length of the queue that forms from time to time?

What is the probability that it will take a customer more than 10 minutes altogether to wait 		for the phone and complete his call?

(OR)

23.	In a railway marshalling yard, goods trains arrive at a rate of 30 trains per day. Assuming that the interarrival time follows an exponential distribution and the service time (the time taken to hump a train) distribution is also exponential with an average of 36 minutes. Calculate:

a.	The Probability that the yard is empty?

b.	Average queuing length, assuming the line capacity of the yard is 9 trains.

24.	Use the graphical method for solving the following and find the value of the game

| Player A/B | B1 | B2 | B3 | B4 | B5 |
| --- | --- | --- | --- | --- | --- |
| A1 | 2 | -1 | 5 | -2 | 6 |
| A2 | -2 | 4 | -3 | 1 | 0 |

(OR)

25.	A production machine installed has initial investment of ` 30, 000 and its salvage value at the end of i years of its use is estimated as ` 30,000/i+1.the annual running cost in the first year is `15000 and increased by `1000 in each subsequent years for first five years and increased by `5000 in each year thereafter. Replacement policy is to be planned over a period of seven years. During this period cost of capital may be taken as 10% per year. Solve the problem for optimum replacement.