Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	AIRCRAFT STRUCTURES								       Time: 3 hours

Subject Code:	09AE202	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define redundancy between a structure.

2.	Skew loads will cause unsymmetrical bending. (True / False)

3.	The locus of the shear centre for all cross sections is called as ________.

4.	A sheet, panel or web which resists only pure shear stress is called as ________.

5.	In a cantilever truss, it is very essential to find out the reactions before analyzing it. (True/False).

6.	A redundant frame is also called ________ frame.

7.	The proof resilience per unit volume of material is known as ________.

8.	The total strain energy stored in a body is known as ________.

9.	The slenderness ratio of long column is ________.

10.	A column of length l is hinged at its both ends. Its equivalent length will be equal to ________.

PART – B (5 x 3 = 15 MARKS)

11.	Differentiate symmetrical and unsymmetrical bending.

12.	What is a Semi cantilever wing? Explain.

13.	Write down the application of Clayperon three-moment equation method.

14.	Define Castiglianos theorem.

15.	Explain briefly Maximum Principal stress theory.

PART – C (5 x 15 = 75 MARKS)

16.	An angle section of Web 20cmx2cm and Flange 20cmx2cm is subjected to a moment of Mx=20kNm and My=50kNm. Find the maximum bending stress.

(OR)

17.	Define shear centre. Determine the shear centre for a C- section beam of Flanges 50mmx2mm and Web 100mmx1mm subjected to a vertical shear load of 1000N.

18.	What is a Tension field web beam? Explain its concept in detail.

(OR)

19.	Discuss in detail the various types of loads acting on the aircraft structure.

20.	A continuous beam ABC 10m long rests on the three supports A, B and C. Determine the moments over the beam and draw the bending moment diagram. Also, calculate the reactions at the supports and draw the shear force diagram. The point A is the extreme left support and the span between A and B is 6m and between B and C is 4m where C is the extreme right support. A point load of 3kN is acting at a distance of 2m from point A and a UDL of 1kN/m is acting between B and C for the whole span of 4m.

(OR)

[P.T.O]

21.	A fixed beam ABC of span 12m carries a point load 12kN and UDL of 2kN/m. Evaluate the bending moment and shear force diagram of the beam. The point A is the left extreme fixed end. The span between the fixed end A and point B is 6M where it is supported. The UDL of 2kN/m is carried over the entire span between Point A and B. The span between B and C is again 6m where C is the extreme right fixed end. The point load of 12kN is acting at a distance of 3m from point C. Use Moment Distribution Method.

22.	A circular cantilever of length l with dia D for l/2 of its length and 2D for the rest of its length carries a load p at the free end. Determine the slope and deflection at the free end.

(OR)

23.	Determine the vertical deflection at point A for the frame shown in figure.

E = 200 kN/mm2  I = 2000cm4

A

B

C

D

E

800mm

800mm

400mm

400mm

12kN

24.	Explain in detail

a.	Maximum strain Theory  	b.	Maximum shear stress Theory 					     (5+10)

(OR)

25.	Derive an expression for buckling load of a column considering one end fixed and other end hinged.