Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	GAS DYNAMICS										       Time: 3 hours

Subject Code:	09AE207	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Speed of sound a = _______.

2.	A choked nozzle is said to be one where the Mach number reaches unity at _______.

3.	Define Shock Wave.

4.	Pressure recovery across a normal shockwave is much higher than a oblique shock wave (True/ False)

5.	The reflection of an incident shock wave from a solid boundary is called as _______.

6.	The small perturbation equations for subsonic and supersonic flows are linear, but for transonic flows the equations is non-linear (True/ False).

7.	Swept back wings are prone to tip stalling (True/ False).

8.	Shadow Graph is a _______ method of flow visualization.

9.	Flow over a Wedge is _______ in nature.

10.	A heat sink is provided in a shock tube or gun tunnel to prevent _______ of the working fluid.

PART – B (5 x 3 = 15 MARKS)

11.	Explain under expanded nozzle.

12.	Define shock polar.

13.	Write the equation for lift and drag at supersonic speeds using linearized theory.

14.	Give the advantages and disadvantages of Swept Wings.

15.	What is a Shock tube? Explain.

PART – C (5 x 15 = 75 MARKS)

16.	Air flows through a frictionless adiabatic convergent divergent nozzle.  The air stagnation pressure and temperature are 7 bar and 500K respectively.  The diverging portion of the nozzle has an area ratio of Aexit/Athroat = 11.91.  A normal shock wave stands in the divergent portion of the nozzle where the Mach number is 3.  Determine the Mach number and the static pressure and temperature at the nozzle exit plane.

(OR)

17.	Derive an expression for Area – Mach number relationship for a C-D nozzle.

18.	Derive an expression for normal shockwave for a perfect Gas.

(OR)

19.	A uniform supersonic airflow and Mach 2 passes over a wedge.  An oblique shock, making an angle of 40 with the flow direction, is attached to the wedge.  If the static pressure and temperature in the free stream are 0.5 bar and 0C, determine the static pressure and temperature behind the wave, the Mach number of the flow passing over the wedge and the wedge angle.

[P.T.O]

20.	What is Prandtl Glauert Theory? Derive an expression for Cp using Prandtl Glauert theory.

(OR)

21.	Derive an expression for Lift and Drag coefficients for an aero foil at supersonic speeds using linearized theory.

22.	Explain in detail:

a.	Transonic Area rule		b.	Critical Mach number   						     (10+5)

(OR)

23.	Discuss in detail the following:

a.	Drag Divergence Mach number		b.	Finite Supersonic Wings			     (5+10)

24.	With a neat sketch explain the working principle of blow down type and Induced flow type wind tunnels.

(OR)

25.	With a neat sketch explain any one method of optical flow visualization.