Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title: 	AIRCRAFT PERFORMANCE							      Time : 3 hours

Subject Code:	09AE216			       					         	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the difference between streamline body and bluff body?

2.	Write the various types of aerofoil.

3.	What is the significance of Drag Polar?

4.	What is camber?

5.	What are the parameters responsible for stalling speed on the aircraft configuration?

6.	What is the significance of zero-lift drag coefficient for minimum drag?

7.	What is the condition for rate of loss of height?

8.	Write the equation angle of glide.

9.	Write the significance of load factor in a turn.

10.	Define experimental mean pitch.

PART – B  (5 x 3 = 15 MARKS)

11.	Define total drag.

12.	What is the significance of induced drag?

13.	Prove that (L/D) /(L/D)max  = 2/ m2 + m-2

14.	What is the importance of assisted take-off?

15.	Write the importance of V-n diagram.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Describe about the aerofoil characteristics.

(OR)

17.	a.	Explain the drag comparison on four different bodies.                        	            		(8)

b.	Write short notes on boundary layer normal pressure drag.                           			(7)

18.	Describe the various methods adopted for reduction of various drags of aeroplane.

(OR)

19.	a.	What is plan form and its various parameters?									(8)

b.	Explain about the pressure distribution on an aerofoil.			               			(7)

20.	An aircraft weighing 250000N has a wing area of 80m2 and its drag equation is

CD= 0.016 +0.04 CL2.  Calculate the minimum thrust required for straight and level flight, and 	the corresponding true air speed at sea-level and at 10,000m  ( √σ =0.58).  Calculate also the 	minimum power required, and the corresponding true air speeds, at the above altitudes.

(OR)

21.	Write short notes on performance curves in terms of thrust and derive the power equation aircraft in level flight.

22.	A sailor weighs 45,000N and has a wing loading of 600 N/m2.  The drag equation CD= 0.700+0.022CL2,  estimate the gliding angle and rate of descent, when driving with air brake, extended at 30 m/sec  at sea level.

(OR)

23.	Derive the equation X = - 1/2a (-B/AVTO2) = 1/2A (VTO2) of take-off ground run.

24.	Derive the equation of minimum time to turn through a given angle for a given aircraft.

(OR)

25.	Derive the equation for efficiency of the disc of propulsive system ηi = 1/1+a