Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	PRINCIPLES OF CHEMICAL ENGINEERING			       Time: 3 hours

Subject Code:	09BT203	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define molecular weight.

2.	Define Molarity.

3.	Define Normality.

4.	Define the term excess reactant.

5.	What is the difference between recycling and purging?

6.	What is friction factor?

7.	Define Reynolds number.

8.	Define fluidization.

9.	What is reciprocating pump?

10.	Classify the various types of pumps.

PART – B (5 x 3 = 15 MARKS)

11.	In an analysis of gas mixture, the following data are obtained: CO2=20%, CO=30%, CH4=40% and H2=10% at atmospheric pressure, calculate the average molecular weight.

12.	An evaporator is used to concentrate cane sugar solutions. A feed of 10,000 kg/day of a solution contains 38% weight solution.  Calculate the weight of solution produced and amount of water removed.

13.	Explain the importance of study of material balance.

14.	What is Fluidization?  Explain the mechanism of fluidization.

15.	Explain the concepts of compressors.

PART – C (5 x 15 = 75 MARKS)

16. 	The waste acid containing 25% HNO3, 50% H2SO4 and 25% H2O by weight .This acid is to be concentrated to contain 30% HNO3 and 60% H2SO4, by the addition of concentrated  H2SO4 containing  95% H2SO4 by weight and concentrated HNO3 containing 90% HNO3 by weight. Calculate the weights of waste acid and concentrated acids to obtain 10,000 kg of mixture of desired composition.

(OR)

17.	A solution of CaCl2 in water contains 450 grams of CaCl2 per liter at 400C. The density of the solution at this temperature is 0.77203 gm/cc. Calculate (a) Composition in weight (b) Vol% of water (c) Composition in mole% (d) Composition in atomic% (e) Molality.

18.	Write the overall material balance equation and component balance equation for distillation and evaporator.

(OR)

19.	1000 kg of sodium carbonate solution containing 23% Na2CO3 is subjected to evaporative cooling.  During which 15% of the water present in the solution is evaporated. From the concentrated solution, Na2CO3.10H2O crystallizes out. Calculate how much crystals would be produced if the solubility Na2CO3 .10H2O be 21.5grams per 100 grams water.

[P.T.O]

20.	Briefly explain about properties of fluids and types of fluids.

(OR)

21.	Write the physical significance of dimensionless numbers (atleast four).

22.	List the various types of valves and explain the any two types of valves.

(OR)

23.	Derive the Ergun equation.

24.	Discuss in detail about characteristics of pumps.

(OR)

25.	Explain the working principles of reciprocating pumps with neat diagram.