Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	REINFORCED CONCRETE STRUCTURES – I			       Time: 3 hours

Subject Code:	09CE210	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

(Use of relevant codes are permitted

USE M20 and Fe 415 steel)

1. 	What is meant by development length?

2.	Give the values of minimum and maximum cover required for tension member.

3.	When and where is torsion reinforcement provided in slab?

4.	Sketch the reinforcement details for a lintel cum sunshade of size 150mm x150mm and width 600mm.

5.	What is the importance of helical reinforcement in columns?

6.	List out the types of shallow foundations.

7.	When do you opt for L-Beam in a structure?

8.	How will you calculate the effective width of T-beam?

9.	Draw the reinforcement detail for a circular slab.

10.	How will you calculate the slenderness ratio of a column with breadth ‘b’ and depth ‘D’?

PART – B (5 x 3 = 15 MARKS)

11.	A rectangular R.C beam of size 250mm x 500mm is reinforced with 3 no of 16mm diameter steel bar in the tension zone. The nominal cover to the reinforcement is 20mm. Determine the position of the actual neutral axis.

12.	What is a lintel? List out the ways in which the loading occur on the lintels.

13.	Calculate the plan area and minimum steel required for a footing to resist an axial load of 500kN.  SBC of soil is 225kN/m2.

14.	A T-beam section has the following details: Breadth and thickness of flange 2000mm and 150mm. Breadth and depth of web is 250mm and 350mm. Tension reinforcement 4-20 dia bars. If the beam is subjected to a design shear of 280kN, check the section for the limit state of collapse in shear.

15.	Differentiate one way slab and two-way slab.

PART – C (5 x 15 = 75 MARKS)

16. 	a.	Explain the design concept of ultimate load method and limit state method.

b.	Explain the various limit states.

(OR)

17.	a.	Draw the actual and idealized stress strain diagram for concrete and steel indicating their 	importance as a whole in a RC structure.

b.	What are the importance of bond and development length in RC structure? Outline the 	codal provisions.

18.	An office building slab of size 5m x 4m is supported on walls.  It is loaded with DL of 5kN/m2 and LL of 3kN/m2. Design the slab. Show the reinforcement details in a neat sketch.

(OR)

[P.T.O]

19.	Design a R.C lintel cum sunshade for a window opening of 1.8m clear span. The height of the wall above the opening is 1.85m and the thickness of the wall is 250mm. The span of sunshade is 550mm. The lintel has a bearing of 250mm on the walls. The imposed load on the sunshade is 800N/m2 and the unit weight of masonry is 19000N/m3.

20.	Design a rectangular combined R.C footing for two columns of size 300mm x 450mm and size 300mm x 300mm carrying an axial load of 800kN and 600kN respectively. The bearing capacity of soil is 150kN/m2.  The centre to centre distance between the columns in 1.5m.

(OR)

21.	A rectangular column of effective length 2.75m is to carry an axial load of 1800kN at an eccentricity of 30mm from its centre. Taking the breadth of column as 300mm x 450mm. Design the column.

22.	A simply supported T-beam has a flange of 1650mm x 110mm size and a web of 300mm x 600mm. It has to resist a design moment of 975kNm. Design the tension reinforcement for the beam.

(OR)

23.	An office building has a roof slab of size 16m x 4 m. The slab is supported on rectangular beams at an interval of 4m centre to centre. Slab is loaded with DL of 5kN/m2 and LL of 3kN/m2. Design a suitable L-beam at the sides to support the slab.

24.	A workshop building is of size 12m x 4.5m. It is loaded with DL of 5kN/m2, LL of 3kN/m2. Design and sketch the reinforcement details of slab.

(OR)

25.	Design a circular column with helical reinforcement to carry an axial load of 1500kN. The effective length of column is 4.5m.