Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	GEOTECHNICAL ENGINEERING						       Time: 3 hours

Subject Code:	09CE212	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define degree of saturation.

2.	Silt particles are visible to the naked eye.  (True / False)

3.	The soil water is classified into _______ and _______.

4.	If the liquidity index of soil is zero.  Find the consistency index.

5.	The delay caused in consolidation by the slow drainage of water out of a saturated soil mass is called _______.

6.	The critical hydraulic gradient of a saturated sand with moisture content of 40% and specific gravity 2.7 is _______.

7.	If the effective stress is maximum the clay is termed as _______.

8.	What is pre-consolidation pressure?

9.	Define stability number.

10.	State Mohr coloumbs equation for determination of shear strength of soil.

PART – B (5 x 3 = 15 MARKS)

11.	What do you understand by residual soil and transported soil?

12. 	Volumetric shrinkage is 90%.  What is the linear shrinkage?

13.	Distinguish between seepage velocity and discharge velocity.

14.	Define Compression index and coefficient of consolidation.

15. 	Find the factor of safety of an infinite slope of cohesionless soil of angle of internal friction 36, if the slope angle is 30.

PART – C (5 x 15 = 75 MARKS)

16. 	A wet soil sample of mass 1.9 kg had a volume of 945 cc.  After oven drying its mass was reduced to 1.7 kg.  The specific gravity of solids was found to be 2.7.  Determine moisture content, bulk density, dry density, void ratio, porosity and degree of saturation.

(OR)

17. 	An earth embankment is to be compacted to a dry density of 1.84g/cc at water content of 15%.  The unit weight and water content in borrow pits are 1.7 g/cc and 8% respectively.  What amount of soil should be carried out in the borrow pit for each cubic meter of the embankment?

18.	How do you distinguish between clay and silt in the field?  State the purpose of the identification and classification of soils.  List any three important engineering classification systems and describe one in detail.

(OR)

19. 	The Atterberg limits of a soil are liquid limit 52%, plastic limit 30% and shrinkage limit 18%.  If the specimen of soil shrinks from a volume of 39.5 cc at the liquid limit to a volume of 24.2 cc at the shrinkage limit, calculate the specific gravity of solids.  Also, classify the soil as per BIS if the fraction passing through 75-micron sieve is 60%.

[P.T.O]

20. 	A sand stratum is 10m thick.  The water table is 2m below the ground level.  The unit weights of sand layer above and below water table are 17kN/ Cu.m and 12kN/Cu.m respectively.  The capillary rise above water table is 1m.  Draw the total stress, pore water pressure and effective stress diagrams for the sand stratum.

(OR)

21. 	Calculate the coefficient of permeability of a soil sample 6 cm in height and 50 sq.cm in cross sectional area, if a quantity of water equal to 432 cc passed down in 10 minutes under an effective constant head of 40cm.  On oven drying, the test specimen weighed 4.98 N.  Taking specific gravity of the solids as 2.65, calculate the seepage velocity of water during the test.

22. 	 A saturated soil has a compression index of 0.25.  Its void ratio at a stress of 10kN/Sq.m is 2.02 and its permeability is 3.4 x 10-7 mm/s.  Compute

a.	Change in void ratio if the stress is increased to 19kN/sq.m						(5)

b.	Settlement if the soil stratum is 5m thick.										(5)

c.	Time required for 40% consolidation if drainage is one way.						(5)

(OR)

23. 	a.	List the assumptions made in one dimensional consolidation theory.				(8)

b.	Explain the procedure for determining preconsolidation pressure.					(7)

24. 	 The following date relate to a Triaxial compression tests performed on a soil sample.

| Test | Chamber pressure kN/sq.m | Max deviator stress kN/sq.m | Pore pressure at max. deviator stress kN/sq.m |
| --- | --- | --- | --- |
| 1 | 80 | 175 | 45 |
| 2 | 150 | 240 | 50 |
| 3 | 210 | 300 | 60 |

Determine the total and effective stress parameters of the soil.

(OR)

25. 	 Explain friction circle method of analyzing the stability of the slopes.