Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DESIGN OF MASONRY AND TIMBER STRUCTURES	       Time: 3 hours

Subject Code:	09CE213	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	How the structures are classified?

2.	Why are codes of practices necessary?

3.	List the classification of walls.

4.	Define shape modification factor.

5.	Define middle third rule.

6.	What is the function of bed block?

7.	Strong bricks and weak mortar are recommended for masonry buildings. Why?

8.	Define the term: bands.

9.	How timber is graded?

10.	What is built up column?

PART – B (5 x 3 = 15 MARKS)

11.	State the advantages of the four basic materials: concrete, timber, masonry and steel.

12.	Explain how permissible stress in masonry is calculated.

13.	What are the general conditions of strength and stability of masonry structures?

14.	State the reasons for the poor performance of masonry buildings in seismic areas.

15.	Mention the factors, which affect the strength of timber.

PART – C (5 x 15 = 75 MARKS)

16.	Distinguish between the working stress method, ultimate method and limit state method of design. What are the features of load and resistant factor design?

(OR)

17.	Explain in detail the various loads acting on structures.

18.	A 200mm thick brick wall carries an axial load of 50kN from walls. Also an eccentric load of 36kN/m from RCC floor slab acting at a distance of 47.5mm from the centre of walls. Determine the stresses developed in the walls.

(OR)

19.	Enumerate the various requirements to be fulfilled by a non-load bearing walls (partition walls). Also, explain the analytical method of designing a masonry wall

20.	A dam 6m high and 1.5m wide at top has vertical water face. Find the base width of the dam if no tension is to develop. Investigate the stability of the dam if the coefficient of friction is 0.6 and the maximum allowable compressive stress is 1800kN/m2. Take unit weight of masonry as 20kN/m3

(OR)

[P.T.O]

21.	Design the foundation for a stone pillar 300mm x 400mm carrying a super imposed load of 300kN at its top. The height of the pillar above ground level is 4m. Unit weight of stone masonry is 22kN/m3 and that of lean cement concrete as 23kN/m3. The soil has an angle of repose of 25o, unit weight of 18kN/m3 and safe bearing capacity of 150kN/m2. The foundation concrete may be in 1:4:8 cement concrete having safe modulus of rupture 245kN/m2.

22.	Describe the various earthquake resistant features that can be introduced in a masonry building to make it earthquake resistant.

(OR)

23.	Describe the behaviour of the following masonry walls in seismic regions.

a.	unreinforced masonry wall  	b.	reinforced masonry wall  and 	c.	infill walls

24.	A timber beam simply supported over an effective span of 6m carries uniformly distributed load of 5.5kN/m inclusive of self-weight. Design a built up beam if it is to be fabricated out of standard grade Sal wood planks of size 50mm x 250mm. Take safe working stress in bending for inside location as 17N/mm2.

(OR)

25.	A built up column of Sal wood consists of four planks of 50mm x 300mm with a central core of 250mm x 250mm. Calculate the safe load carrying capacity if its height is 4m. Take

fcp = 10.6N/mm2 and E = 12700N/mm2.