Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	APPLIED HYDRAULICS AND FLUID MACHINES  		       Time: 3 hours

Subject Code:	09CE216	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Distinguish between uniform flow and Non-uniform flow.

2.	Write down Kutter’s equation used in open channel flow.

3.	Momentum thickness (θ) is given by ________.

4.	Total drag on the body is the sum of ________ and ________.

5.	The force exerted by the jet of water on a stationery plate in the direction of jet is ________for an inclined plate.

6.	Define the term impact of jets.

7.	What is priming?

8.	Define slip in a reciprocating pump.

9.	Distinguish between an impulse turbine and reaction turbine.

10.	Draft tube is installed in ________turbine.

PART – B (5 x 3 = 15 MARKS)

11.	Explain the terms rapidly varied flow and gradually varied flow.

12.	What is meant by boundary layer? Why does it increase with distance from the upstream edge?

13.	Find the force exerted by the jet of water of diameter 100 mm on a stationary plate, when the jet strikes the plate normally with a velocity of 30 m/sec.

14.	Explain briefly about governing of turbines.

15.	Explain briefly the working principle of air vessel in a reciprocating pump.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain briefly the various types of flow in an open channel.

b.	Find the bed slope of a trapezoidal channel of bed width of 3 m from and the slope of 3 horizontal to 4 vertical, when the discharge through the side of the channel is 30m3/sec. Take chezy’s constant as 70.

(OR)

17.	a.	What is specific energy Curve? Draw the specific energy curve and derive the expression 	for critical depth and critical velocity.

b.	The discharge of water through a rectangular channel of width 8m, is 15 m3/sec. When the depth of the flow of water is 1.2 m. Calculate

i)	Specific energy of the flowing water  	ii)	Critical depth and critical velocity

iii)	Value of minimum specific energy.

18.	a.	Explain briefly about the boundary layer separation and highlight about the methods of 	preventing separation of boundary layer.

b.	Find the displacement thickness and energy thickness for a velocity distribution in         	the boundary layer given by u/u= 2 (y/δ) – (y/δ)2

(OR)

[P.T.O]

19.	a.	Derive an expression for drag and lift.

b.	Experiments were conducted in a wind tunnel with a speed of 50 km/ hr on a flat plate of size 2 m long and 1 m wide. The density of air is 1.15 kg/m3. The coefficient of lift and drag are 0.75 and 0.15 respectively, determine lift force, drag force, resultant force and direction of resultant force.

20.	a.	Derive an expression for a force exerted by the curved plate at the centre of the plate 	when 	moving along the direction of jet.

b.	A jet of water of diameter 50 mm moving with a velocity of 40m/s strikes a curved            fixed symmetrical plate at the centre. Find the force excreted by the jet of water in the         direction of jet, if the jet is deflected through an angle of 120º at the outlet of the curved         plate.

(OR)

21.	a.	Derive an expression for a force exerted by a jet on a stationary vertical plate.

b.	A jet of water of diameter 75mm moving with a velocity of 25m/s strikes a fixed plate in   	such a way that the angle between the jet and the plate is 60 º. Find the force exerted by the 	jet on the plate in the direction normal to the plate, in the direction of flow.

22.	A reaction turbine works at 450 rpm under a head of 120 m, its diameter at inlet is 120 cm and the flow area is 0.4 m2. The angles made by the absolute and relative velocities at inlet are 20º and 60º respectively with a tangential velocity 30m/s.  Determine the volume rate of flow, power developed and hydraulic efficiency assuming whirl at the outlet to be zero.

(OR)

23.	A pelton wheel turbine is to be designed for the following specifications, Power = 9500kw, Head = 350 m, speed = 750 rpm, overall efficiencies = 85%, jet diameter 1/6th of the wheel diameter. Take CN =0.987, speed ratio =0.45. Determine

a.	Wheel diameter, 		b.	Diameter of the jet 		c.	No of jets required.

24.	The centrifugal pump has the following characteristics. Outer diameter of the impellor is        800 mm, width of the impellor vane at the outlet is 400 mm, angle of the impellor vane at       the outlet is 40º, the impellor runs at a speed of 550 rpm and discharges 0.98 m3/s under an effective head of 35 m. A 500 kw motor is used to drive the pump. Determine manometric, mechanical and overall efficiency of the pump; assume water enters the impeller vanes radially at the inlet.

(OR)

25.	Draw a neat sketch of a submersible pump and explain the operating principles of the submersible pump.