Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	REINFORCED CONCRETE STRUCTURES – II  		       Time: 3 hours

Subject Code:	09CE217	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Bring out the difference between rise and tread in stair case.

2.	Mention the types of retaining wall (any two).

3.	Circular water tank is preferable than rectangular water for the given capacity. (True / False)

4.	What is meant by free board?

5.	Name any two types of IRC loading classification.

6.	Mention the components of a Tee beam bridge.

7.	Reinforced concrete multistorey building frames are statically determinate structures.  (True / False)

8.	Name any two methods of analysis used for analyzing the reinforced concrete multistorey building frames.

9.	The yield lines are analogous to the plastic hinge referred to in the inelastic analysis of beams and frames.  (True / False)

10.	Name the two theories available for the analysis of two-way slab under inelastic condition.

PART – B (5 x 3 = 15 MARKS)

11.	Specify the different types of staircases and draw the sketch of doglegged staircase.

12.	Mention the critical condition to be considered in the design of underground water tank.

13.	What are the conditions to be satisfied for using Courbon’s method in the analysis and design of Tee beam and slab decks?

14.	Write down the assumptions made in the cantilever method.

15.	List out the assumptions made in the yield line theory.

PART – C (5 x 15 = 75 MARKS)

16.	Design one of the flights of a doglegged staircase of multi-stored building using the following data:

Number of steps (risers) in the Going = 10

Tread (T) = 300 mm, Rise (R) = 150 mm

Width of landing slab on either side = 1 m

Materials: M20 grade concrete and Fe-415 HYSD bars.

(OR)

17.	Design the stem portion of a cantilever retaining wall to retain an earth embankment 4 m high above ground level.  The density of earth is 18 kN/m2 and its angle of repose is 30°.  The embankment is horizontal at its top.  The safe bearing capacity of the soil is 200 kN/m3 and the coefficient of friction between the soil and concrete is 0.5.  Adopt M20 grade and Fe-415 HYSD bars.  Check the stability condition also.

[P.T.O]

18.	Design an R.C.C. circular water tank resting on the ground with a flexible base and a spherical dome for storing 500000 litres of water.  The depth of storage is to be 4 m.  Free board = 200 mm.  Adopt M20 grade concrete and Fe-415 HYSD bars.  Permissible stresses should comply with the values recommended in IS: 456 – 2000 codes.

(OR)

19.	Design an underground reservoir 12 m x 5 m x 4 m deep.  The angle of repose of soil is 30° and density of soil is 16,000 N/m3.  Use M20 grade of concrete and Fe -415 bars.

20.	Design a reinforced concrete slab culvert for a state highway to sit the following data:

Carriageway – Two lane (7.5 m wide)

Materials – M20 grade and Fe-415 bars

Kerbs – 600 mm wide

Clear span = 6 m, Wearing coat = 80 mm

Width of bearing = 0.4 m

Loading – I.R.C. Class A or AA whichever gives the worst effect.  Design the R.C. deck slab and sketch the details of reinforcements in the longitudinal and cross-sections of the slab.

(OR)

21.	Design the interior slab panel of a R.C.C. Tee beam girder bridge to suit the following data:

Clear width of road way = 7.5 m

Span (centre to centre of bearings) = 16 m

Live load – I.R.C. Class AA or A whichever gives the worst effect

Average thickness of wearing coat = 80 mm

Materials – M20 grade and Fe-415 bars

22.	Analyse the building frame, subjected to horizontal forces as shown in Figure below.  Use portal method.

120 kN

180 kN

A

B

C

D

E

F

G

H

I

J

K

L

7 m

3.5 m

5 m

3.5 m

3.5 m

(OR)

23.	Analyse the frame, Figure shown in Problem 22 by cantilever method, assuming that all the columns have the same area of cross-section.

24.	Write short notes on the following:													(3x5)

a.	Concept of yield line theory		b.	Yield line pattern for square and rectangular slab

c.	Yield moments.

(OR)

25.	Design a rectangular slab 7.5 m by 5 m simply supported on all edges.  The live load is 6000 N/m2.  The load factors are 1.5 and 2.2 for dead load and live load respectively.  The ultimate moment in the longer direction is half of the shorter side.  Take fy = 250 N/m2 and fck = 16 N/mm2.