Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DESIGN OF STEEL STRUCTURES (LIMIT STATE METHOD)

Time: 3 hours

Subject Code:	09CE219	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are the types of structural steel sections?

2.	Define pitch.

3.	Mention the failures caused in tension members.

4.	What is the need of gusset plate?

5.	Differentiate between primary beam and secondary beam.

6.	Define “laterally unrestrained”.

7.	What are the types of column bases?

8.	What is a plate girder?

9.	Give the applications of GI sheets. Mention the commercial sizes available in the market.

10.	What is a “bay”?

PART – B (5 x 3 = 15 MARKS)

11.	Explain the classifications of bolt.

12.	Write short notes on splices.

13.	What is torsional bracing?

14.	What are the elements of plate girder?

15.	List the load acting on the trusses.

PART – C (5 x 15 = 75 MARKS)

16.	The plates of a water tank are 8 mm thick, connected by single bolt and nut lap joint using 20mm diameter bolt at 50 mm pitch. Calculate the efficiency of the joint.

(OR)

17.	A tie member of a roof truss consists of 2 ISA 100X75X8 mm. The angles are connected either side of a 10 mm gusset plate and member is subjected to a working pull of 300 kN. Design the weld connection. Assume workshop weld.

18.	Determine the tensile strength of a roof truss diagonal 100X75X6 mm (fy=250 Mpa) connected to a gusset plate.

(OR)

19.	Design a laced column 10m long to carry factored axial load of 1000 kN. The column is restrained in position but not direction at both ends.

20.	Determine the design bending capacity of ISLB 350 @ 0.486 kN/m, the beam is laterally unsupported.  The unsupported length of the beam is 4m.  Assume fy = 250 N/mm2.

(OR)

21.	Design a simply supported beam of effective span 3m laterally unsupported.  The beam carries a factored UDL 50kN/m throughout the span.

[P.T.O]

22.	Design a welded plate girder of span 18m, carrying a superimposed load of 25kN/m.  Assume fy = 250N/mm2.  The plate girder doesn’t have bearing/ intermediate stiffeners.

(OR)

23.	Design a slab base of column ISHB 300 carrying an axial factored load of 1000kN. Use M20 grade concrete.

24.	Design suitable angle purlin on a sloping roof with dead load=0.15kN/m2, Live Load = 2 kN/m2, Wind load= 0.5kN/m2. The purlins are spaced at 2 m center to center of span 4m simply supported on rafter slope 20 degree.

(OR)

25.	Design a flink type roof truss for an industrial building with the following data. The industry is situated in Naini, Allahabad. Assume suitably the missing data.

Overall length			= 48 m

Overall width			= 16.5 m

Width c/c of roof columns	= 16m

Height of columns		= 11m

Roofing material			= Asbestos cement sheet

Side covering			= Asbestos cement sheet