Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	FOUNDATION ENGINEERING   						       Time: 3 hours

Subject Code:	09CE221       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	List the various approximate methods of stress determination in soil mass.

2.	Name the limitations of plate load test.

3.	What type of samplers can be used to collect undisturbed samples?

4.	Define Standard Penetration Number ‘N’.

5.	What is a compensated raft foundation?

6.	Draw a neat sketch of an under reamed pile.

7.	What is Earth Pressure at rest?

8.	What are the assumptions in Rankine’s theory of Earth pressure?

9.	Define optimum moisture content.

10.	What are the various methods of soil compaction?

PART – B (5 x 3 = 15 MARKS)

11.	Explain the various factors influencing bearing capacity of soil.

12.	What is the principle of seismic refraction method of soil investigation?

13.	What is negative skin friction?

14.	How do you determine the height of unsupported cut in soils?

15.	What is field compaction control?

PART – C (5 x 15 = 75 MARKS)

16. 	Explain the construction and use of New marks Influence chart.

(OR)

17.	Compute the safe bearing capacity of a square footing 1.5 mx1.5m, located at a depth of lm below ground level in  sand of average density 20 kN/m3 Φ=20,    Nc=17.7, Nq=7.4 Nr=50. Adopt a factor of safety =3 and assume that the water table is very deep. If the water table rises to the ground level, find the reduction in the safe bearing capacity.

18.	Explain standard penetration test with corrections.

(OR)

19.	Explain various methods of boring with neat sketches.

20.	A pile group consists of 9 piles of 30 cm diameter and 10 m length driven in clay. Unconfined  compressive  strength  of  soil  is  200  kN/m2  and  insitu density of soil   is    20 kN/m3. Determine the safe load for the pile group.  = 0.6 and F.S. = 3.

(OR)

21.	When do you go for a combined footing? What are the various types of combined footing? How do you proportion a combined footing?

[P.T.O]

22.	Compute the intensities of active and passive earth pressure at depth of 8m in dry   cohesion less sand with an angle of internal friction of 30o and unit weight of 18kN/m3.  What will be the intensities of active and passive earth pressure if the water level rises to the ground level. Take saturated unit weight of sand as 22 kN/m3.

(OR)

23.	Explain how do you check the stability of a retaining wall.

24.	Explain the different methods of soil stabilization.

(OR)

25.	Explain various field compaction tests.