Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	FLUID MECHANICS AND MACHINERY    				       Time: 3 hours

Subject Code:	09CE242	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Write the relationship between dynamic viscosity and kinematic viscosity?

2.	What is the centre of pressure?

3.	Write the continuity equation for three-dimensional flows.

4.	What is streamline?

5.	What are the purposes of orifice?

6.	Name the different types of notches.

7.	Write the impulse momentum equation.

8.	Draw the ideal indicator diagram for a reciprocating pump.

9.	How would you classify turbines based on the direction of flow in the runner?

10.	 Draft tube is required for _________ turbines.

PART – B (5 x 3 = 15 MARKS)

11.	Define ‘Surface tension’.

12.	Define ‘Stream function’.

13.	What do you mean by ‘Water hammer’?

14.	What do you mean by ‘cavitations’ in centrifugal pump?

15.	Define ‘Specific speed’ of a turbine.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Name the equipments used to measure the pressure.									(5)

b.	A rectangular plate 3m long and 2m wide is immersed vertically in water in such that its 3m side is parallel to the water surface and is 2m below it. Find: (i) Total pressure on the plate and (ii) Position of centre of pressure.													(10)

(OR)

17.	a.	Derive an expression for the capillary rise.											(5)

b.	A vertical cylinder of diameter 200mm rotates concentrically inside another cylinder of diameter 201.2mm. Both the cylinders are 300mm high. The space between the cylinders is filled with a liquid whose viscosity is unknown. Determine the viscosity of the fluid if a torque of 15Nm is required to rotate the inner cylinder at 100rpm.										(10)

18.	a.	Derive the continuity equation for three-dimensional flow.								(8)

b.	In a two dimensional incompressible flow, the fluid velocity components are given by u=x – 4y and v= -y – 4x. Show that velocity potential exists and determine its form as well as stream function.																				(7)

(OR)

19.	a.	In a smooth inclined pipe of uniform diameter 250mm, a pressure of 50kN/m2 was 	observed at 	section 1 which was at elevation 10m. At another section 2 at elevation 12m, 	the pressure was 	20kN/m2 and the velocity was 1.25m/s. Determine the direction of flow 	and the head loss between 	these two sections. The density of water at 20°C is 998kg/m3.							(7)

b.	Derive the Bernoulli’s equation for a streamline flow from the Euler’s equation.			(8)

[P.T.O]

20.	a.	A submarine fitted with a pitot tube moves horizontally in sea. Its axis is 12m below the 	surface of water. The pitot tube fixed in front of the sub-marine and along its axis is 	connected 	to the two limbs of a U-tube containing mercury, the reading of which is found 	to be 200mm. 	Find the speed of the sub-marine. Take the specific gravity of seawater as1.025.			(6)

b.	A horizontal venturimeter with inlet diameter 200mm and throat diameter 100mm is employed to measure the flow of water. The pressure at inlet is 0.18N/mm2 and the vacuum pressure at the throat is 280mm mercury. Find the rate of flow. The value of coefficient of discharge may be taken as 0.98.				   														(9)

(OR)

21.	a.	A rectangular notch has a discharge of 0.36m3/s, when head of water is 0.75m. Find the 	length of 	the notch. Assume coefficient of discharge as 0.6.	   								(6)

b.	Three pipes of diameters 300mm, 200mm and 400mm and lengths 450m, 255m and 315m respectively are connected in series. The difference in water surface levels in two tanks is 18m. determine the rate of flow of water if coefficients of friction are 0.0075, 0.0078 and 0.0072 respectively considering: (i) Minor losses also and (ii) neglecting minor losses.			(9)

22.	a.	Define (i) manometric efficiency and (ii) Overall efficiency of a centrifugal pump. 			(3)

b.	The impeller of a centrifugal pump has an external diameter of 450mm and internal diameter of 200mm. The speed of the pump is 1440rpm. Assuming a constant radial flow through the impeller at 2.5m/s and that the vanes at exit are set back at an angle of 25°, Determine: (i) the inlet vane angle, (ii) the angle, the absolute velocity of water at exit makes with the tangent and (iii) the work done per unit weight.															(12)

(OR)

23.	a.	Explain the working principle of double acting reciprocating pump with a neat sketch.		(7)

b.	A single acting reciprocating pump has a plunger diameter of 250mm and stroke length of 350mm. The speed of the pump is 60rpm and the discharge is 0.02cumecs of water. Determine: (i) the theoretical discharge, (ii) coefficient of discharge (iii) percentage slip.					(8)

24.	a.	Distinguish between impulse and reaction turbines.									(6)

b.	A Pelton wheel is required to develop 9000 kW when working under the head of 300m. The speed of the Pelton wheel is 540rpm. The coefficient of velocity is 0.98 and the speed ratio is 0.46. Assuming jet ratio as 10 and overall efficiency as 84%, Determine: (i) the number of jets, (ii) the diameter of the wheel and (iii) the quantity of water required.							(9)

(OR)

25.	a.	Write notes on governing of turbines. 												(5)

b.	A Francis turbine is to be designed to develop 360kW under a head of 70m and a speed of 750rpm. The ratio of width of runner to diameter of runner ‘n’ is 0.1. The inner diameter of the runner is half the outer diameter. The flow ratio is 0.15. The hydraulic efficiency is 95% and the mechanical efficiency is 84%. Five percent of the circumferential area of runner is to be occupied by the thickness of the vanes. The velocity of flow is constant and the discharge is radial at exit. Determine: (i) the diameter of the wheel, (ii) the quantity of water supplied, (iii) the guide vane angle at inlet and (iv) runner vane angles at inlet and exit.								(10)