Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTROMAGNETIC FIELDS							       Time: 3 hours

Subject Code:	09EC221	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Magnetic monopoles do exist. (True / False)

2.	Polarization is _______ per unit volume.

3.	Curl of an irrotational field is _______.

4.	Cartesian, Cylindrical and _______ coordinate systems are used to analyze electric and magnetic fields.

5.	The current produced by bound charges is called Amperian current. (True / False)

6.	Unit of mmf-“ampere-turns”.  (True / False)

7.	The divergence of the vector flux density is the outflow of flux from a small closed surface per unit volume as the volume shrinks to _______.

8.	If the divergence is zero in an area, there is neither a _______ nor sink in that area.

9.	No work is involved in moving a unit charge around an _______ surface.

10.	_______ found that electric current can affect compass needle.

PART – B (5 x 3 = 15 MARKS)

11.	Transform a vector A=y Ix−x Iy+z Iz into cylindrical co–ordinates.

12.	State Gauss's law for the magnetic field.

13.	Define self-inductance and mutual inductance.

14.	What is meant by hysteresis?  Draw the hysteresis loop.

15.	Define dielectic strength.

PART – C (5 x 15 = 75 MARKS)

16.	Derive an expression for the Electric field intensity at a point P due to an Electric dipole. Hence, define Electric dipole moment.

(OR)

17.	a.	State Divergence Theorem.													(3)

b.	A vector field D=5r24 Ir is given in spherical co–ordinates.  Evaluate both sides of divergence theorem for the volume enclosed between r = 1 and r = 2.				(12)

18.	Derive Maxwell's curl equations from Ampere's law and Faraday's law.  Express the equations in phasor form for time harmonic fields.

(OR)

19.	A cylindrical conductor of radius a and permeability μ carries a Z-directed, uniformly distributed current I. Evaluate the magnetization M, within the conductor.

[P.T.O]

20.	Assuming static conditions derive the boundary relations for Magnetic fields.

(OR)

21.	Derive the relationship between B, H and M in a linear, isotropic, homogeneous media. Express Ampere’s circuital law using the several current.

22.	Assuming static conditions derive the boundary relations for electric fields.

(OR)

23.	Solve one-dimensional Laplace equation to obtain the field inside a parallel plate capacitor, and also find the expression for the surface charge density at two plates.

24.	Write down Maxwell's equation in integral form and point form for time varying fields.

(OR)

25.	Derive the Poynting Vector from Maxwell's equation and explain.