Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DIGITAL SIGNAL PROCESSING						       Time: 3 hours

Subject Code:	09EC224	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Determine the DTFT of the sequence x[n]=an u[n].

2.	Calculate the number of complex multiplication for direct evaluation of 8-point DFT.

3.	What is the reason that FIR filter is always stable?

4.	Write the N-point Bartlett windows equation.

5.	What is locus of the poles of Chebyshev filter?

6.	Find the number of delays required in a Direct Form-I that realizes the third order transfer function.

7.	Mention the limit cycles that occur in Digital Signal Processing.

8.	Draw the quantization noise model for a second order IIR system.

9.	What are the different stages in pipelining?

10.	What is Widrow –Hoff equation?

PART – B (5 x 3 = 15 MARKS)

11.	Find Linear convolution of x(n)={1,2,3} and h(n)={1,1 } via circular convolution.

12.	What are Gibbs oscillations?

13.	Is bilinear transformation linear or not? What is the merit and demerit of bilinear transformation?

14.	Why rounding is preferred to truncation in realizing digital filter?

15.	Differentiate between Von Neumann and Harvard architectures.

PART – C (5 x 15 = 75 MARKS)

16. 	a.	Compute 4 point DFT of a sequence x[n] ={ 0,1,2,3} using DIT and DIF algorithms 	(10)

b.	Find IDFT for X[k]= {4,0,0,0} using DIT algorithm.								(5)

(OR)

17.	Discuss in detail about the time response of discrete time systems for standard input signals.

18.  Find the output of the FIR filter whose impulse response is h[n]={1,2} and the input signal x[n]={1,2,-1,2,3,-2,-3,-1,1,1,2,-1},using overlap save method.

(OR)

19.	The desired frequency response of a low pass filter is given by Hd(ejw) = e−j2π; for−π4≤w≤π4

a.	Determine the filter coefficients hd[n]

b.	Obtain the coefficients h[n] of a 5-tap filter based on the window method with a 	rectangular window.

c.	Find the frequency response and draw the linear phase realization structure of the filter.

[P.T.O]

20.	A Low pass filter has desired responseHd(ω) = e−j3π: for−π2≤w≤π2.

a.	Determine the filter coefficients and transfer function for N=7 using frequency sampling 			technique.

b.	Draw the Direct form realization structure.

(OR)

21.	Design a Butterworth filter satisfying the following specifications: 0.707≤ |H(ejw) | ≤ 1 for 0≤w≤π2|H(ejw) | ≤ 0.2 for 3π4≤w≤π

Assume T=1 sec. Apply bilinear transformation method.

22.	a.	Explain the characteristics of a limit cycle oscillation with respect to the system described 	by the difference equation y[n] =0.95y [n-1] + x[n].Determine the dead band of the filter. 	Consider five bits (including the sign bit) for the quantization process. 				(10)

b.	Draw the product quantization noise model of second order IIR system	.			(5)

(OR)

23.	a.	The coefficients of a system defined by H (z) = 1/ (1-0.4Z-1) (1-0.55Z-1) are represented in 	number with a sign bit and 3 data bits. Determine the new pole locations for (i).Direct 	realization and (ii) cascade realization of first order systems. Compare the movements of 	the new poles away from the original ones in both the case.						(10)

b.	List the different methods to avoid adder overflow.								(5)

24.	a.	In relation to DSP processor, explain the following techniques: (In each case clearly point 	out the advantages and disadvantages of the technique in signal processing)

SIMD,	VLIW.																(8)

b.	A multiplier-accumulator, with three pipe stages, is required for a digital signal processing. Sketch a block diagram of a suitable configuration for MAC with the aid of a timing diagram, explain how the MAC works.											(7)

(OR)

25.	With neat flowchart, explain about LMS algorithm used for adaptive filtering. List out the practical limitations of basic LMS.