Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MICROPROCESSORS AND MICROCONTROLLERS	       Time: 3 hours

Subject Code:	09EC233	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is word length of a microprocessor?

2.	Define: cross assembler.

3.	What is called minimum and maximum mode in 8086?

4.	Differentiate Intel 8086 and 8088 microprocessors.

5.	Mention the need for interfacing devices in a system.

6.	Expand USART.

7.	What is called watchdog timer in Intel 8031?

8.	Mention the need for the flash type memory in microcontrollers.

9.	What are called SFRs in Intel 8031?

10.	Compare counters and Timers.

PART – B (5 x 3 = 15 MARKS)

11.	Define and explain: Machine cycles, T-states, Instruction cycles.

12.	What is called memory segmentation in 8086? Briefly, Discuss in detail.

13.	By drawing the control word of Intel 8255, explain it in detail.

14.	Explain the input and out port structure of Intel 8051.

15.	Explain the need for serial interfacing circuits in microcontrollers by mentioning any one important applications of it.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Design an interfacing circuit using a 3 x 8 decoder to interface.  Explain memory chip 	(4kx8) so that the memory address of this chip ranges from 2000 to 2FFFH.			(8)

b.	Discuss the various stages of evolution of microprocessor in detail.	            			(7)

(OR)

17.	Explain the classifications of instructions set and addressing modes of Intel 8085.  Give two examples for each.

18.	By drawing a neat block diagram, explain the architecture of Intel 8086 microprocessor. Also, explain the details of the internal registers structure.

(OR)

19.	Write an assembly language program in Intel 8086 for arranging the given numbers in the ascending order by providing the necessary flowchart and algorithm.

20.	Explain the features and functions of programmable interrupt controller and discuss the expansion mode of the same.

(OR)

21.	Explain how the keyboard and display devices are interfaced with the processors using the Intel 8279 by providing the necessary block diagrams and control word formats.

[P.T.O]

22.	Explain how the addressing modes are framed in Intel 8031 and how are they related to each classification of Instructions.

(OR)

23.	Write an assembly language program in Intel 8051 for Sum of series of ten numbers by providing the necessary flowchart and algorithm.

24.	Briefly explain the need for interrupt structure of Intel 8031 by explaining all the available interrupts in it.

(OR)

25.	Explain the following with the neat block diagram and flowchart:

a.	LCD Interfacing		b.	ADC and Sensor Interfacing circuits.