Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	BASIC ELECTRICAL ENGINEERING 					      Time: 3 hours

Subject Code:	09EE101	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The resistance offered by 1m length of the wire of material having an area of cross section of 1m2 is called as _________.

2.	 One horse Power = _________Watts.

3.	Reluctance S= _________A / Wb.

4.	Any induced emf will circulate a current in such a direction so as to oppose the cause producing it is called as _________.

5.	The primary fuel for nuclear reactor is _________.

6.	The cosine of the Phase angle between the voltage and current is called as_________.

7.	The induced emf that opposes the voltage applied to the armature of a dc motor is called as _________.

8.	An alternator works on the principle of _________.

9.	_________ damping is produced in moving coil instruments.

10.	In _________type of wiring a single lamp is controlled from two places.

PART – B (5 x 3 = 15 MARKS)

11.	State Kirchoff’s Laws.

12.	What is mutually induced emf?

13.	Define average value of a sinusoidal voltage and give its formula.

14.	Discuss the principle of operation of a Transformer.

15.	What is the necessity to provide earthing?

PART – C (5 x 15 = 75 MARKS)

16.	Explain briefly about current division technique for the following:

a.	Two resistances are connected in Parallel.                     				       			(8)

b.	Three resistances are connected in Parallel.                     				       			(7)

(OR)

17.	Derive the expression for the equivalent star network resistances for a given delta network.

18.	a.	Compare magnetic circuits and electric circuits.					     				(10)

b.	Write any five characteristics of magnetic flux lines.				      				(5)

(OR)

19.	Explain the concepts of mutual inductance and self inductance and define the units in which these are measured.  Derive the expression for coefficient of coupling in electric circuits.

20.	With a neat sketch explain the operation of Thermal power plant.

(OR)

21.	A resistance of 100 Ω is connected in series with a 50µf capacitor to a supply of 200V, 50 Hz. Find the (i) Impedance, current, power factor and phase angle  (ii) the voltage across resistor and capacitor. Draw the phasor diagram also.

[P.T.O]

22.	Explain the principle of operation of a DC motor and also derive the expression for armature torque.

(OR)

23.	Discuss briefly about the construction and principle of operation of a three phase Induction Motor with a neat diagram.

24.	With a neat sketch explain the principle of operation of various types of moving iron instruments.

(OR)

25.	a.	Describe briefly about fluorescent lamp with its wiring circuit.                                	(10)

b.	Write short notes on switches.							  						(5)