Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	COMMUNICATION ENGINEERING					       Time: 3 hours

Subject Code:	09EE203	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Amplitude modulation index.

2.	Mention the frequency range of microwave.

3.	What is quantization error?

4.	Which type of pulse modulation is called digital modulation?

5.	What is coherent detection?

6.	What is binary phase shift keying?

7.	What is refraction?

8.	Mention any one advantage of optical communication.

9.	What is aspect ratio?

10.	What are the basic colours used in colour television?

PART – B (5 x 3 = 15 MARKS)

11.	The two modulation indices of AM wave given as m1 = 0.6 and m2=0.8, what is total modulation index.

12.	Mention merits and demerits of delta modulation.

13.	Explain the method of generation of BPSK signal.

14.	What are the characteristics of cable?

15.	What is scanning process?

PART – C (5 x 15 = 75 MARKS)

16. 	a.	Differentiate the linear and nonlinear AM modulators.							(5)

b.	Explain the collector AM modulator with circuit diagram.							(10)

(OR)

17.	a.	Explain the reactance FM modulator using FET.									(8)

b.	A frequency modulated signal given by efm(t)= 12sin(6x106t+ 5 sin 1250t).  Find carrier 	frequency, modulating frequency, modulation index, and bandwidth.				(7)

18.	Explain modulation and demodulation process in pulse code modulation.

(OR)

19.	a.	Explain the generation and detection of pulse amplitude modulation in detail.		(8)

b.	Compare various pulse modulation schemes.										(7)

20.	Explain the principle of amplitude shift keying and derive the relation for error probability of 	binary ASK.

(OR)

21.	a.	Compare BPSK and DPSK.                                                                                      	(5)

b.	Explain coherent detection of BPSK signal.										(10)

[P.T.O]

22.	Draw the block diagram of optical communication system and explain its operation in detail.

(OR)

23.	What is multiplexing?  Explain the concept of FDM and TDM with appropriate diagrams.  Also 	compare FDM and TDM.

24.	Explain TV transmission and reception briefly with a neat block diagram.

(OR)

25.	Explain PAL decoder in colour television.