Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	SIGNALS AND SYSTEMS								       Time: 3 hours

Subject Code:	09EI218		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are the different types of representation of discrete time signals?

2.	List the properties of convolution.

3.	Define Fourier series.

4.	The Fourier transform of e-atu(t) is _______.

5.	What is the Nyquist sampling rate of the signal x(t)=sinc2(100πt)?

6.	Define anti-aliasing filter.

7.	The N point DFT of a sequence x(n) is _______.

8.	What is the sufficient condition for the existence of DTFT?

9.	Define inverse Z-transform.

10.	The Z transform f(z) of the function F(nT) = anT is _______.

PART – B (5 x 3 = 15 MARKS)

11.	Determine the response of the relaxed system characterized by the impulse response              h(n) = (1/3)nu(n) to the input signal x(n) = 2nu(n).

12.	State Parseval’s theorem for continuous time Fourier series.

13.	What is aperture effect? How can it be reduced?

14.	State DTFS for a periodic signal x(n).

15.	State initial and final value theorem of z-transform.

PART – C (5 x 15 = 75 MARKS)

16.	Determine the step response of the system described by the difference equation.

y(n) + 4y(n-1) + 4y(n-2) = x(n)

(OR)

17.	Find the convolution of   x(n) = u(n) - 2u(n-3) + u(n-6) with  h(n) = u(n+2) - u(n-9).

18.	a.	Explain the conditions under which any periodic waveform can be expressed using Fourier 	series.				                                                                       					(3)

b.	State different properties of Fourier series.										(12)

(OR)

19.	Find the Fourier series for the periodic signal x(t) = t, 0 ≤ t ≤ 1 and repeats every 1sec.

20.	Explain the reconstruction of band limited signals from its samples. Derive the necessary equations and draw the waveforms.

(OR)

21.	a.	State sampling theorem. Also discuss aliasing by taking an example.	          		(7)

b.	The signal x(t) = 10cos(10πt) is sampled at a rate 8 samples per second. Plot the amplitude 	spectrum for │Ω│ ≤ 30π. Can the original signal be recovered from samples? Explain.

(8)

[P.T.O]

22.	State and prove any four properties of discrete time Fourier transform.

(OR)

23.	Determine and sketch the magnitude and phase response of the given system.

y(n) = 1/3[x(n) + x(n-1) + x(n-2)]

24.	Find the output of the system whose input and the output are related by

y(n) = 7y(n-1) – 12y(n-2) + 2x(n) – x(n-2) for the input x(n) = u(n).

(OR)

25.	Find the inverse z-transform of the following.

a.	z2/[(1-az)(z-a)]			b.	1/[1+z-1+z-2]								           	(7+8)