Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MICROPROCESSORS AND MICROCONTROLLERS	       Time: 3 hours

Subject Code:	09EI222		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	How many flags are supported by 8085 Microprocessor?

2.	Indicate any two control signals of 8085 Microprocessor.

3.	What type of addressing mode is supported by IN 80H instruction?

4.	Draw the instruction format of three-byte instructions.

5.	Specify the operating modes of 8255 IC.

6.	How are interrupts initiated in the microprocessor environment?

7.	What is the need of DMA transfer?

8.	Mention the internal ROM and RAM size of 8051 microcontroller.

9.	How many external interrupts are supported by 8051 microcontroller?

10.	How many modes are supported by 8051 timers?

PART – B (5 x 3 = 15 MARKS)

11.	Draw the flag register of 8085 processor and indicate its bit position.

12.	Define the term machine cycle and instruction cycle.

13.	Compare memory mapped I/O and I/O mapped I/O.

14.	Mention the important features of DMA controller.

15.	Specify any three addressing modes of 8051 with suitable example.

PART – C (5 x 15 = 75 MARKS)

16. 	Explain the functional block diagram of 8085 microprocessor in detail.

(OR)

17.	Draw and explain the timing diagram of STA 8000H instruction of 8085 processor in detail.

18.	a.	Explain the various addressing modes of 8085 Microprocessor with an example.     	(8)

b.	Explain any five arithmetic instructions of 8085 Microprocessor with an example.  	(7)

(OR)

19.	Write an assembly language programme for performing multi byte (2 bytes) addition of two given numbers using 8085 processor.

20.	With neat diagram, explain the operation and operating modes of 8255 interface in detail.

(OR)

21.	Explain, how keyboards and LED displays are interfaced to microprocessors in detail.

22.	Write short notes on:

a.	Interrupt structure and its servicing methods    	b.	Development of ISR’s 	     (10+5)

(OR)

23.	Explain the interfacing concepts of stepper motor and its control in detail.

[P.T.O]

24.	Explain the architectural diagram of 8051 microcontroller in detail.

(OR)

25.	Explain the different operating modes of serial interface of 8051 controller in detail.