Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title : 	COMPUTER ARCHITECTURE							      Time : 3 hours

Subject Code:	09IT201			       						             	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is hardwired program?

2.	Number of blocks in main memory is ------------------------ if number of bits used to address a word is ‘n’ and the number of words in a fixed block is ‘K’?

3.	State drawbacks of Interrupt-driven I/O.

4.	What is SDRAM? How is it different from DRAM?

5.	Compute (1.0000000010 x 25) – (1.0000000000 x 2-2) and normalize the results.

6.	State the difference between arithmetic shift and logical shift.

7.	State the usage of branch history table in pipelining.

8.	A pipelined processor with 5 stages executes a program with 10000 instructions which are issued at a rate of one/clock cycle. What is the speedup factor of this processor compared to non-pipelined processor?

9.	Give an example of a horizontal microinstruction.

10.	Show the micro-operations for ‘Indirect Cycle’.

PART – B  (5 x 3 = 15 MARKS)

11.	Write about the performance parameters of memory.

12.	Consider a machine with byte addressable main memory of 232 bytes and block size of 16 bytes. Assume a set associative cache consisting of 4K lines divided into two-line sets. How is the address divided into tag, set and byte number?

13.	Perform two’s complement integer multiplication for   (-7) x (3)

14.	Write a short note on interrupt vector table in Pentium processor

15.	Write short notes on Wilkes control

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss with neat diagram the top level view of computer components and detail about the basic instruction cycle.

(OR)

17.	Explain in detail about cache read operation and direct mapping function.

18.	How will you detect and correct single bit error in 8-bit words using Hamming error-correcting code. Explain using a 8-bit error data.

(OR)

19.	What is programmed I/O? Explain the role of the processor in programmed I/O using relevant block diagrams.

(OR)

20.	Draw and explain block diagram of unsigned binary division and perform the calculation for ‘(0110)/(0010)’.

(OR)

21.	Explain in detail about the most common transfer-of control operations found in instruction sets.

[P.T.O]

22.	a.	Briefly discuss about instruction pipelining with 6 pipeline stages (FI, DI, CO, FO, EI,

WO).  If the 5th instruction is a ‘conditional branch’ instruction and jump to 20th instruction in memory, explain the effect of conditional branch instruction using timing diagram.

(12+3)

b.	State the different approaches to deal with conditional branches in pipelining processors

(OR)

23.	Explain in detail about register organization in a general computer.

24.	Explain the Hardware Implementation of Control Unit with an example.

(OR)

25.	Describe about micro instruction sequencing in detail. What are the address generating techniques?