Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MECHANICS OF MACHINES - I						       Time: 3 hours

Subject Code:	09ME205	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Attachment of a car mirror is an example of _______ pair.

2.	The component of acceleration, parallel to the velocity of the particle at a given instant is _______ component.

3.	The velocity of the belt for transmission of maximum power is _______.

4.	Name two materials used for lining of brakes.

5.	The size of a cam depends upon_______.

6.	What is an oscillating follower?

7.	What is a reverted gear train?

8.	In a gear train, if the number of idler gears is odd, then the rotation of the driven gear will be _______ as that of the driving gear.

9.	What is a gyroscope?

10.	When the speed of the governor increases, the radius of rotation _______

PART – B (5 x 3 = 15 MARKS)

11.	Define degrees of freedom. Find the number of degrees of freedom of the mechanism shown in figure.

C

D

B

A

12.	Differentiate between hydrostatic and hydrodynamic bearings.

13.	A cam raises a roller follower moving with simple harmonic motion. The follower has a stroke of 50 mm. The outstroke angle is 120˚ and the return stroke angle is 60˚. Find the maximum velocity and acceleration of the follower during outstroke.

14.	A pair of 20˚ involute spur gears having 30 and 50 teeth respectively of module 4 mm is in mesh. In order to avoid interference, find the addendum of the smaller gear.

15.	Define the terms (a) Sensitiveness (b) Stability and (c) Hunting of a governor.

PART – C (5 x 15 = 75 MARKS)

16. 	The crank of a slider crank mechanism rotates clockwise at a constant speed of 300 rpm. The crank is 150 mm and connecting rod is 600 mm long. Determine (i) linear velocity and acceleration of the	midpoint of connecting rod and (ii) angular velocity and angular acceleration of the connecting	rod, at a crank angle of 45˚ from the inner dead centre position.

(OR)

[P.T.O]

P

Q

R

S

62.5 mm

175 mm

112.5 mm

10 rad/s

200 mm

17.	PQRS is a four bar chain with link PS fixed as shown in figure. The lengths of the links are PQ = 62.5 mm; QR = 175 mm; RS = 112.5 mm and PS = 200 mm. The crank PQ rotates at 10 rad/s clockwise. Angle QPS = 60˚ and Q and R lie on the same side of PS. Find the angular velocity and angular acceleration of the links QR and RS.

18.	a.	A leather belt transmits 7.5 kW from a 1.2 m diameter pulley, running at 250 rpm. Angle 	embraced is 165˚ and µ = 0.3. Safe working stress for the leather belt is 1.5 MPa; density 	of leather is 1 Mg/m3 and thickness of the belt is 10 mm. Determine the width of the belt, 	taking centrifugal tension into account.											(8)

b.	A single plate clutch, effective on both sides transmits 25 kW at 3000 rpm. Determine the dimensions of the frictional surface if the coefficient of friction are 0.255; the ratio of radii is 1.25 and the maximum pressure is not to exceed 0.1 N/mm2. Assume theory of uniform wear.																		(7)

(OR)

19.	a.	A rope drive transmits 600 kW from a pulley of effective diameter 4 m, which runs at 90 	rpm. Angle of lap is 160˚; Angle of the groove is 45˚; coefficient of friction is 0.28; the 	mass of the rope is 1.5 kg/m and the allowable tension in each rope is 2400 N. Find the 	number of ropes required.														(8)

b.	A multiple disc clutch has five plates having four pairs of active friction surfaces. The intensity of pressure is 0.127 N/mm2. Find the power transmitted at 500 rpm. The outer and inner radii of friction surfaces are 125 mm and 75 mm respectively. Take coefficient of friction as 0.3.																(7)

20.	A cam with a minimum radius of 25 mm is to be designed for a knife-edge follower with the following data:

*	To raise the follower through 35 mm during 60˚ of cam rotation

*	Dwell for the next 40˚ of cam rotation

*	Follower descends during next 90˚ of cam rotation

*	Dwell for the rest of cam rotation

Draw the profile of the cam if it raises the follower with simple harmonic motion and the follower descends with uniform velocity. The line of stroke of the follower is offset 10 mm from the axis of the camshaft.

(OR)

21.	Draw the profile of a cam operating a roller follower with the following date:

Minimum radius of the cam = 25 mm; Lift = 40 mm; Roller diameter = 15 mm.

The cam lifts the follower for 120˚ with simple harmonic motion followed by a dwell period of 30˚. Then the follower lowers down during 150˚ of the cam rotation with uniform acceleration and deceleration, followed by a dwell period.

22.	A pair of 20˚ involute gears are in mesh. Module = 6 mm; Number of teeth on pinion = 17; Number of teeth on gear = 49; Speed of pinion = 1500 rpm. Addendum on pinion and gear wheel is 1 module. Find (a) The number of pairs of teeth in contact; (b) Angle turned through by gear wheel when one pair of teeth are in contact and (c) Sliding velocity at point of engagement and disengagement.

(OR)

[P.T.O]

23.	An epicyclic gear train, as shown in figure, has a fixed annular wheel A having 150 teeth. The wheel A meshes with wheel B, which drives wheel D through the idle wheel C. D is concentric with A. The wheels B and C are carried on an arm, which revolves clockwise at 100 rpm about the axis of A and D. Wheels B and D have 25 teeth and 40 teeth respectively.  Find the number of teeth on C and the speed of rotation of C.

A

D

C

B

Arm

24.	A Porter governor has all four arms 250 mm long. The upper arms are attached on the axis of rotation and the lower arms are attached to the sleeve at a distance of 30 mm from the axis. The mass of each ball is 5 kg and the sleeve has a mass of 50 kg. The extreme radii of rotation are 150 mm and 200 mm. Find the range of speed of the governor.

(OR)

25.	The turbine rotor of a ship has a mass of 2000 kg and rotates at a speed of 3000 rpm clockwise when looking from the stern. The radius of gyration of the rotor is 0.5 m. Determine the gyroscopic couple and its effect, when the ship steers to the right in a curve of 100 m radius at a speed of 29.88 km/hr.  Also find the gyroscopic couple and its effect when the ship is pitching in simple harmonic motion, the bow falling with its maximum velocity. The period of pitching is 50 seconds and total angular displacement between the extreme positions is 12˚. Also find the maximum acceleration during pitching.