Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	HEAT ENGINES AND FLUID MACHINERY			       Time: 3 hours

Subject Code:	09ME228	       						               	       Maximum Marks: 100

#### Answer ALL questions

(Approved data books may be permitted)

PART – A (10 x 1 = 10 MARKS)

1.	Calculate the pressure in Pascal’s when it is measured in terms of

a.	Water column of 20 cm		b.	Mercury column of 5 cm

2.	Define cavitation and causes for creating cavitation.

3.	How do you differentiate between impulse and reaction turbine?

4.	What is the principle of operation of a rotating machine and what factor governs the energy transfer?

5.	What is quasi-static process? What is its characteristics feature?

6.	PMM2 is impossible. (True/False)

7.	What is an air standard cycle? Why are such cycles conceived?

8.	List out the methods to determine frictional power of an IC engine.

9.	What do you mean by thermal conductivity? What do you mean by insulator?

10.	State Stefan Boltzmann law.

PART – B (5 x 3 = 15 MARKS)

11.	A differential manometer is connected to two pipes whose centres are at 3 m difference in height. Higher level pipe is carrying liquid of specific gravity of 0.9 at a pressure of 1.8 bar and another pipe is carrying liquid at specific gravity of 1.5 at a pressure of 1 bar. The centre of pipe carrying low pressure liquid is 2 m above the higher level of the mercury in the manometer. Find out the difference in mercury level in the manometer in cm.

12.	Differentiate between inward radial flow and outward radial flow reaction turbines. Why inward flow reaction turbines are preferred in practice?

13.	12 kg of air per minute is delivered by a centrifugal air compressor. The inlet and outlet conditions of air V1 = 12 m/s, p1 = 1 bar, v1 = 0.5 m3/kg and V2 = 90 m/s, p2 = 8 bar, v2 = 0.14 m3/kg. The increase in enthalpy of air passing through the compressor is 150 kJ/kg and heat loss to the surroundings is 700 kJ/min. Determine motor power required to drive the compressor and ratio of inlet to outlet pipe diameter.

14.	Write a short note on “Air standard analysis” which has been adopted for IC engine cycles. State the assumption made for air standard cycles.

15.	Consider two identical bodies, one at 1000 K and the other at 1500 K. Which body emits more radiation at a wavelength of 20 µm?

PART – C (5 x 15 = 75 MARKS)

16.	The inner and outer diameters of a centrifugal pump are 15 cm and 25 cm. It delivers 50 L/s when running at 1500 rpm. The velocity of flow through the impeller is constant at 2.5 m/s. The vane angle is 30o. The suction and delivery pipes diameters are 15 cm and 10 cm respectively. The pressure head at suction is 4 m below and at delivery is 18 m above atmosphere. Find inlet vane angle, manometric efficiency and overall efficiency.  Take atmospheric pressure head = 10.33 m of water and power required is 18.5 kW.

(OR)

[P.T.O]

17.	a.	Two pipelines A and B as shown in figure carrying oils of specific gravity of 1.5 and 0.9 	respectively. A U-tube manometer is used to measure the pressure difference using 	mercury as manometric liquid. Find the pressure difference in kPa.

b.	A converging pipe is 20 cm inlet and 10 cm outlet and 5 m long is lying in the vertical 	plane making an angle 45o to the horizontal. The pipe is carrying the water 24 m3/min. The 	pressure of the water at inlet is 500 kPa, find the pressure of the water at the exit.

18.	A Kaplan turbine is running at 80 rpm when head available is 10 m. The vane angle at the extreme edge of the runner is 15o. TakingVf2gH=0.6 and DbDo=0.4, find the diameter of the boss and diameter of the runner. Also find discharge through the runner. Assume flow velocity is constant through the runner and discharge is radial.

(OR)

19.	An inward flow reaction turbine working under a head of 200 m has inlet and outlet diameters 4 m and 2.5 m respectively. The absolute velocity at exit is 18 m/s and whirl velocity is zero. The absolute velocity at inlet makes an angle of 30o with peripheral velocity. Assuming the width of the runner is constant throughout, find, a) hydraulic efficiency of the turbine, b) RPM of the runner, c) relative velocities at inlet and outlet.

20.	A gas undergoes a thermodynamic cycle consisting of the following processes:

a.	Process 1-2: Constant pressure p = 1.4 bar, V1 = 0.028 m3, W12 = 10.5 kJ,

b.	Process 2-3: Compression with pV = constant, U3= U2, (c) Process 3-1: Constant volume, 	U1 – U3 = -26.4 kJ. There are no significant changes in KE and PE.

i)	Sketch the cycle on p-V diagram, 	(ii)	Calculate the net work for the cycle in kJ,

iii)	Calculate the heat transfer for process 1-2, and

iv)	Show that CycleW=cycleQ.

(OR)

21.	A gas flows steadily through a rotary compressor. The gas enters the compressor at a temperature of 16oC, a pressure of 100 kPa, and an enthalpy of 391.2 kJ/kg. The gas leaves the compressor at a temperature of 245oC, a pressure of 0.6 MPa, and an enthalpy of 534.5 kJ/kg. There is no transfer of heat to or from the gas as it flows through the compressor, (i) Evaluate the external work done per unit mass of gas assuming the gas velocities at entry and exit to be negligible, (ii) Evaluate the external work done per unit mass of gas when the gas velocity at entry is 80 m/s and that at exit is 160 m/s.

22.	In an air standard Diesel cycle, the compression ratio is 16, and at the beginning of isentropic compression, the temperature is 15oC and the pressure is 0.1 Mpa. Heat is added until the temperature at the end of the constant pressure process is 1480oC. Calculate (a) the cut-off ratio, (b) the heat supplied per kg of air, and (c) the cycle efficiency.

(OR)

[P.T.O]

23.	In a gas turbine plant working on the Brayton cycle the air at inlet is at 27oC, 0.1 MPa. The pressure ratio is 6.25 and the maximum temperature is 800oC. The turbine and compressor efficiencies are each 80%. Find (a) the compressor work per kg of air, (b) the turbine work per kg of air, (c) the heat supplied per kg of air, (d) the cycle efficiency, and (e) the turbine exhaust temperature.

24.	a.	Why is the heat transfer coefficient for natural convection is much less that for forced 	convection.

b.	Explain why fins are commonly used and explain why fins are generally used in a gas side of gas to liquid heat exchangers.

c.	Differentiate between steady and unsteady state heat transfer.

(OR)

25.	The wall of cold storage consists of three layers-an outer layer of ordinary bricks, 25 cm thick, a middle layer of cork, 10 cm thick and inner layer of cement, 6 cm thick. The thermal conductivities of the materials are 0.7, 0.043 and 0.72 W/m K, respectively. The temperature of the outer surface of the wall 30oC and that of inner is -15oC. Calculate:

a.	Steady state rate of heat gain per unit area.

b.	Temperature at the interfaces of composite wall.

c.	The percentage of total heat resistance offered by individual layers.