Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	POWER PLANT ENGINEERING						       Time: 3 hours

Subject Code:	09ME237	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	State any two methods of increasing the efficiency of Rankine cycle.

2.	Name the working fluid used in binary vapour cycle.

3.	How is dust collectors classified?

4.	Write any two requirements of coal handling systems.

5.	What do you understand by nuclear fission?

6.	List the materials used for gas turbine blades.

7.	Write the formula to find brake thermal efficiency and brake power in diesel engine.

8.	Define water hammer.

9.	What is meant by the term tariff in power plant economics?

10.	Write any two functions of solar collector.

PART – B (5 x 3 = 15 MARKS)

11.	Sketch and label the line diagram for a binary vapour cycle.

12.	What are the advantages of modern high pressure boilers?

13.	Enumerate the advantages of nuclear power plants.

14.	State the factors to be considered in selecting a hydraulic turbine.

15.	Differentiate load curve and load duration curve.

PART – C (5 x 15 = 75 MARKS)

16.	In a Rankine cycle, the steam at inlet to turbine is saturated at a pressure of 30 bar and the exhaust pressure is 0.2 bar. Determine:

a.	Pump work		b.	The turbine work       c.	Cycle efficiency

d.	Dryness at the end of expansion           		e.	Condenser heat flow

(OR)

17.	A steam power plant operates on a theoretical reheat cycle. Steam in the boiler at 150 bar, 550℃ expands through the high pressure turbine. It is reheated at a constant pressure of 40 bar and 550 ℃ and expands through the high-pressure turbine to a condenser at 0.1 bar. Find

a.	Quality of steam at turbine exhaust    	b.	Cycle efficiency	 c.	Steam rate in kg/kwh

18.	a.	Discuss the water and steam flow circuit in a steam power plant.                     		(10)

b.	Sketch and explain the pneumatic ash handling system.                                     		(5)

(OR)

19.	a.	With a neat diagram, explain the travelling grate stoker firing of coal.                		(8)

b.	Briefly write about the super critical boilers.                                                       		(7)

20.	a.	Explain the working of fast breeder reactor with a neat diagram.                        		(10)

b.	Discuss the salient features of nuclear waste disposal.                                          		(5)

(OR)

[P.T.O]

21.	a.	Enumerate the differences between an open cycle and a closed cycle gas turbine power 	plant. 								               								(5)

b.	With a neat layout, discuss the functions of reheater, regenerator and intercooler in a      	gas turbine power plant. 								 						(10)

22.	Sketch and explain the layout of the diesel power plant.

(OR)

23.	Explain with a neat diagram, the working of a pumped storage hydro power plant. Also list its major applications and advantages.

24.	The yearly duration of curve of a certain plant can be considered as a straight line from 300 MW to 80 MW. Power is supplied with one generating unit of 200 MW capacity and two units of 100 MW capacity each. Determine: 												(5x3)

a.	Installed capacity		b.	Load factor		c.	Plant factor		d.	Maximum demand

e.	Utilization factor

(OR)

25.	a.	Explain with a neat diagram, working of a wind power plant.                            		(8)

b.	Discuss the two-pool power plant with a suitable sketch.                                   	 	(7)