Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ADVANCED DIGITAL SIGNAL PROCESSING 			       Time: 3 hours

Subject Code:	10EI305			         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Mention the various classifications of signals.  Show that any signal can be decomposed 	into odd and even components. Illustrate this with an example.						(10)

b.	Obtain the impulse response h(n) for the system described by the second order difference 	equation  y(n)  – 3y(n-1) - 4y(n-2) = x(n) +2x(n-1).								(10)

2.	a.	Formulate the steps involved in the decimation in frequency FFT algorithm. Mention the 	advantages of FFT over direct computation of DFT.								(10)

b.	Compute the N point DFT of x(n) = an   for 0 ≤ n ≤ N-1.							(10)

(OR)

3.	a.	Compute the 8 point DFT of the given sequence using the flowgraph of Decimation in time 	FFT:    x(n)  = { 1,3,2,2,1,3,2,2}												(10)

b.	Determine the inverse Z transform of X(z)  = z−4+z−2(z−12)(z−14).					(10)

4.	a.	Obtain the direct form and cascade realization for the IIR system described by the 	difference equation  y(n) + y(n-1) -1/4  y(n-2)  = x(n)	.							(10)

b.	Explain the characteristic features of Butterworth filters. Design an analog low pass 	Butterworth filter to satisfy the specifications:  pass band frequency 1600 π rad/sec, 	stop 	band frequency 3200π rad/sec, pass band attenuation 0.5dB, stop band attenuation 45	dB.

(10)

(OR)

5.	a.	Describe how mapping between s and z plane is carried out using Bilinear transformation 	method and discuss the advantages of this method.								(10)

b.	Design a digital Butterworth low pass IIR filter using Bilinear transformation method for the following specifications :  sampling time 1sec,  passband frequency 0.06π rad/sec, stopband frequency 0.75π rad/sec, passband attenuation 6dB,  stopband attenuation 20dB.

(10)

6.	a.	Mention the desirable features of the various windows used in FIR filter design.		(10)

b.	Design a low pass filter using Hanning window with cut off frequency of 0.9 rad/sec and 	choosing N = 6.																(10)

(OR)

7.	a.	Determine the coefficients of a linear phase FIR filter with a cut off frequency of π/2   			rad/sec using frequency sampling method, assuming N = 11.						(10)

b.	Describe the principle of equiripple linear phase FIR filters and their design using 	MATLAB.																	(10)

[P.T.O]

8.	a.	Describe with relevant equations, the process of decimation by a factor D and obtain the 	spectra of the decimated signal.												(10)

b.	Describe the use of polyphase structures for the sampling rate converters.			(10)

(OR)

9.	a.	Describe the method of sampling rate conversion by a rational factor I/D and obtain the 	equation of the spectrum of the output sequence.									(10)

b.	Describe multistage implementation of sampling rate conversion.					(10)