Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title: 	ADVANCED STRENGTH OF MATERIALS				       Time: 3 hours

Subject Code:	10ME301		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Briefly explain the mohr’s circles for the three dimensional state of stress. 			(10)

b.	Derive the differential equations of equilibrium. 									(10)

2.	Determine the diameter ‘d’ of a circular shaft subjected to a bending moment ‘m’ and a torque ‘t’, according to the following failure theories. Use factor of safety ‘N’					(4x5)

a.	Maximum normal stress theory		b.	Maximum shear stress theory

c.	Maximum strain theory			d.	Maximum energy theory

(OR)

3.	a.	Determine the diameter of a cold-rolled steel shaft, 0.6m long, used to transmit 50 hp at 	600 rpm. The shaft is simply supported at its ends in bearings. The shaft experiences 	bending owing to its own weight also. Use a factor of safety 2. The tensile yield limit is 	280x103 kpa and the shear yield limit is 140x103 kpa. Use the maximum shear stress 	theory.																		(10)

b.	Briefly explain the yield surfaces of Tresca and Von mises.						(10)

4.	Explain the first and second theorem of Castingliano.

(OR)

5.	a.	Two conditions must be satisfied by an ideal piston ring. (i) It should be truly circular 	when in the cylinder, and (ii) it should exert a uniform pressure all around. Assuming that 	these conditions are satisfied by specifying the initial shape and the cross-section show that 	the initial gap width must be 3πpr4/EI, if the ring is closed inside the cylinder. P is the 	uniform pressure per centimeter of circumference. EI is kept constant.				(12)

b.	A cantilever is supported at the free end by an elastic spring of spring constant k. Determine the reaction at fixed end. The cantilever beam is uniformly loaded. The intensity of loading is W.														(8)

6.	a.	Derive the equations for torsion of thin-walled tubes.								(10)

b.	Derive the equations for torsion of rectangular bars.								(10)

(OR)

7.	a.	A cantilever beam of rectangular section (4cm x 6cm) is subjected to a load of 1000N 	which is inclined at an angle of 30° to the vertical. What is the stress due to bending at 	fixed point D near the built-in-end?												(8)

b.	Derive the winkler-Bach formula for bending of curved beams.					(12)

8.	Derive the equations of airy stress function and Prandtl’s stress function for torsion.

(OR)

9.	Briefly discuss about contact stresses and stress concentrations in theory of elasticity.