Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	FINITE ELEMENT ANALYSIS 							       Time: 3 hours

Subject Code:	10ME302		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Describe the general applicability of finite element method for solving structural, thermal, 	fluid flow and coupled field problems with examples. 							(10)

b.	Explain the principle of Choleski method for the solution of equilibrium problems with an 	illustrative example. 															(10)

2.	a.	Explain in detail the discretization process in FEM.								(12)

b.	Derive the shape functions and body force vector for spar element. 					(8)

(OR)

3.	Determine the mid-span displacement for a steel beam of diameter 12 mm and length 1m which is fixed at both ends. The beam is subjected to a transverse point load of 6kN at the mid-span of the beam.  Model the beam using two elements and take Young’s modulus as 210 GPa.

4.	Each of the three bars of the pin-jointed frame shown in figure has a cross-sectional area of 1000 mm2 with E = 200 GPa.    Solve for the displacements and the reaction forces.

(OR)

5.	A vertical plate of thickness 25 mm is tapered with widths of 150 mm and 130mm at its top and bottom ends respectively. The plate is fixed at the top end. The length of the plate is 500 mm. Take Young’s modulus as 200 GPa and density as 7200 kg/m3. Model the plate with two spar elements and determine the natural frequencies of longitudinal vibration.

6.	A triangular plate of thickness 8 mm has vertices P(50,50), Q(100,50) and R(70,120). It is fixed at P and supported on rollers at Q. There is a vertical downward load of 5 kN applied at R. Take Young’s modulus as 200 GPa. Determine the nodal displacements, element strain, element stress and the reaction forces.

(OR)

7.	Establish the shape functions and derive the strain displacement matrix for a rectangular element.

8.	Establish the finite element equations for solving fluid flow problems using Galerkin approach.

(OR)

9.	determine the temperature distribution in a steel rod using three finite elements.  The rod is attached to a structure at a temperature of 600°C. The rod is of length 500 mm, diameter 8 mm and thermal conductivity 70 W/m°C. The rod is dissipating heat by convection into ambient air at 32°C with a heat transfer coefficient of 18 W/m2°C.