Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title : 		OPERATIONS MANAGEMENT				             Time : 3 hours

Subject Code: 		10MS311				         				       Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Consider the manufacture of a toy. The master Production Schedule to manufacture the toy is given below

| Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Demand | 200 | - | 100 | 175 | 300 | 200 | - | 250 |

The product structure is given below

A

B

C

D

E

The details of Bill of Materials along with economic order quantity and stock in Hand for the final product and subassemblies are shown in the following table.

| Part Required | Order Quantity | No of Units | Lead Time ( Week ) | Stock in Hand |
| --- | --- | --- | --- | --- |
| A | 350 | 1 | 2 | 200 |
| B | 450 | 1 | 1 | 400 |
| C | 400 | 1 | 1 | 375 |
| D | 375 | 1 | 1 | 250 |
| E | 400 | 1 | 2 | 425 |

Complete the Materials Requirement plan for the main product A as well as for the sub assemblies B, C, D and E.

2.	a.	With suitable examples, discuss the various types of Production system.  					(10)

b.	Describe the various elements of an Operations Strategy.                     					(10)

(OR)

3.	The following is the sales data for the past three years of Dord Limited. Dord Limited is a leading manufacturer of auto components. The Factory manager wants to forecast the Quarter wise sales for the next year. Select a suitable forecasting technique and forecast the sales for the four quarters of the next year.

[P.T.O]

|  | Quarterly sales ( in thousands of units ) |  |  |  | Annual Total sales |
| --- | --- | --- | --- | --- | --- |
|  | Q1 | Q2 | Q3 | Q4 |  |
| 8 | 520 | 730 | 820 | 530 | 2600 |
| 9 | 590 | 810 | 900 | 600 | 2900 |
| 10 ( Last year) | 650 | 900 | 1000 | 650 | 3200 |
| Total | 1760 | 2440 | 2720 | 1780 | 8700 |

4.	“Chennai is a leading hub for Auto and Auto components manufacturing in India “ - Comment

(OR)

5.	The following is the demand forecast for a group of itmes of Baruthi Limited. The production planning and control department has decided the follow the following mixed strategy for the aggregate plan, namely 1) Maintain a constant production rate of 250 per quarter. 2) Permit 20 % Over Time when the demand exceeds the production rate. The incremental cost of Overtime is Rs 25 per unit.  3) To meet any further demand, the department decides to hire and fire workers.  The department also estimates that it costs Rs 150 per unit to increase the production rate, Rs 200 per unit to decrease the production rate. Rs 50 per unit per quarter to carry the items on inventory and Rs 100 if subcontracted.

| Quarter | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Demand | 270 | 220 | 470 | 670 | 450 | 270 | 200 | 370 |

Perform the cost calculation for the strategy adopted for the aggregate plan.

6.	a.	Discuss the seven Tools of Quality Control                            						(10)

b.	Discuss the key elements of Total Quality Management       						(10)

(OR)

7.	From the below data calculate the control limits. Plot the control charts and comment on the same.

| Date | June 06 | 07 | 08 | 09 | 11 | 12 | 13 | 14 | 15 | 16 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Numberi nspected | 3350 | 3354 | 1509 | 2190 | 2678 | 3252 | 4641 | 3782 | 2993 | 3382 | 3694 |
| Number rejected | 31 | 113 | 28 | 20 | 35 | 68 | 339 | 12 | 3 | 17 | 14 |
| Date | 19 | 20 | 21 | 22 | 23 | 25 | 26 | 27 | 28 | 29 | 30 |
| Number inspected | 3052 | 3477 | 4051 | 3042 | 1623 | 915 | 1644 | 1572 | 1961 | 2440 | 2086 |
| Number rejected | 8 | 27 | 44 | 70 | 12 | 9 | 1 | 22 | 3 | 3 | 1 |

8.	a.	Write a note on Total Productive Maintenance                            					(10)

b.	Discuss the key elements of Supply chain Management              					(10)

(OR)

9.	A manufacturing plant must develop a cost estimate for a customer’s order for eight large turbine shafts. It is estimated that the first shaft will take 100 hours of shop time, and an 80 percent learning curve is expected. 																(4 X 5 = 20)

a.	How many labour hours should the eighth shaft require?

b.	How many labour hours should the whole order for eight shafts require?

c.	If the labour rate is Rs 19.00 per hour and the pricing policy of the company is to double

the labour cost of the order, what is the customer price for each shaft?

d.	Trouble is encountered on the order, and it is obvious that the original estimate was low because it took 90 labour hours for the third shaft. The company wants to approach the customer with a revised price for the order. What should the new customer price be for the whole order?