Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MASS TRANSFER OPERATIONS						       Time: 3 hours

Subject Code:	BT236		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	When Henry’s law constant is very small, mass transfer is controlled by _______.

2.	Diffusivity of vapor is determined in a laboratory by _______.

3.	With increase in reflux ratio, the number of plate in the binary distillation unit _______.

4.	The system Carbon disulphide – Acetone is an example for _______.

5.	Liquid redistributors are used in tall packed tower to _______.

6.	Separating agents for extractive and azeotropic distillation are normally _______.

7.	Adsorption is an operation, which generally involves _______.

8.	Freundlich equation is applicable to _______.

9.	For leaching vegetable seeds, extractors used are _______ and _______.

10.	In Bollmann extractor the flow is _______ and _______.

PART – B (5 x 3 = 15 MARKS)

11.	Differentiate molecular diffusion from eddy diffusion.

12.	How will you define the effect of feed condition?

13.	On what basis the selection of solvent for absorption is done?

14.	Differentiate physical adsorption from chemisorptions.

15.	Discuss the properties of solvent for extraction?

PART – C (5 x 15 = 75 MARKS)

16. 	a.	Derive from fundamental the expression for steady state diffusion of gas ‘A’ through a non 	- diffusing gas ‘B’								  							(10)

b.	Calculate the rate of diffusion of Acetic acid ‘A’ across a film of non-diffusing H2O ‘B’ solution 2 mm thick at 17°C, when the concentration (by weight) on opposite sides of the film are 10% and 4% acids. The diffusivity of acetic acid in the solution is 9.5 x 10-5 m2/s            and the density of 10% and 4% acids (by weight) are 1013 kg/m3 and 1004 kg/m3            respectively.										   							(5)

(OR)

17.	a.	Derive the relationship between individual and overall mass transfer. For a gas-liquid 	system, when is the system said to be gas film or liquid film controlled? 		  	(10)

b.	What is inter phase mass transfer? Explain two-phase mass transfer concept? 		(5)

18.	a.	Derive Rayleigh equation for differential distillation.				   				(5)

b.	A liquid feed at the boiling point of 400 Kg mole / hr containing 70-mole % of benzene and 30 mole % toluene is fed to a stripping tower at 103.3 K Pa pressure. The bottom product flow is to be 60 kg mole / hr containing only 10-mole % of benzene and the rest is toluene. Reflux ratio is 4.0.  	            			 			  	  				(10)

| x | 0 | 0.1 | 0.2 | 0.3 | 0.4 | 0.5 | 0.6 | 0.7 | 0.8 | 0.9 | 1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| y | 0 | 0.2 | 0.38 | 0.51 | 0.63 | 0.71 | 0.78 | 0.85 | 0.91 | 0.96 | 1.0 |

i)	Calculate kg mole / hr of overhead product and its composition.

ii)	Determine the number of theoretical stages.

(OR)

[P.T.O]

19.	a.	Write down the steps involved in constructing the number of ideal stages using McCabe 	and Theile method for continuous distillation column.				    				(7)

b.	Write a note on azeotropic distillation and extractive distillation.		    			(8)

20.	a.	An ammonia-air mixture containing 2 % by volume ammonia is to be scrubbed with water 	at 20oC in a tower packed with 1.27 cm Raschig rings. The water and gas rates are 1170 kg 	/ hr m2 each, based on empty tower cross section. Estimate the height of the tower required 	if 98 % of the ammonia in the entering gas is to be absorbed. The tower operates at 1 atm 	pressure. The equilibrium relationship is given by the following equation.  y = 0.746 x. 	Where y = mole fraction of ammonia in air, x = mole fraction of ammonia in solution with 	water. The height of transfer unit may be taken as equal to 2 meters.	          		(10)

b.	List out the steps involved in constructing operating line and equilibrium curve for an 	absorber and stripper.    			   			    		   						(5)

(OR)

21.	a.	Derive an expression to determine the height of an absorption column based on condition 	of gas film.			    						  								(10)

b.	How will you determine the tray efficiency for absorption tower? Explain.	   		(5)

22.	a.	Explain the principle and applications of Higgins contactor moving bed absorber with neat 	diagram.				    						  								(10)

b.	Compare the Freundlich and Langmuir equation, which represents equilibrium adsorption 	data.									   									(5)

(OR)

23.	a.	Explain the principle and applications of any one industrially used ion exchange 	equipments.	      									  							(10)

b.	Write a note on fixed bed adsorber.             					   					(5)

24.	a.	Discuss the principle and application of Bollmann extractor used in leaching operation 	with neat diagram.    									  						(10)

b.	What is the effect of temperature on ternary liquid equilibria? Explain.            		(5)

(OR)

25.	Explain the packed extraction tower and pulse column extractor with neat diagram. Also list out their industrial applications.