Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ENGINEERING MECHANICS							       Time: 3 hours

Subject Code:	CE204		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The force acting on a particle is ______.

2.	Differentiate resolution of forces and composition of forces.

3.	The types of supports are ______.

4.	What is friction?

5.	The polar moment of inertia for the shaft is ______.

6.	What is centroid?

7.	In curvilinear motion, the direction of the instantaneous velocity is always ______.

8.	______ represents the rate of change in the velocity of a particle.

9.	Energy is measured in ______.

10.	The impulse experienced by an object is the force x ______.

PART – B (5 x 3 = 15 MARKS)

11.	Write the parallelogram law of forces.

12.	Distinguish between positive and negative couple.

13.	What is radius of gyration? Distinguish moment of inertia and polar moment of inertia.

14.	Define rectilinear motion of a particle.

15.	If you drop a 5-gram water balloon from 2 meters above the earth, how will you calculate the gravitational potential energy?

PART – C (5 x 15 = 75 MARKS)

16.	A bar AB hinged to the foundation at A and supported by a strut CD is subjected to a horizontal 50 kN load at B as shown in Figure.  Determine the nature and magnitude of force in the strut and also the reaction at A.

(OR)

17.	Five strings are fixed at a point and are pulled in all directions, equally spaced from one another. If the magnitude of the pulls on three consecutive strings is 50N, 70N and 60N respectively, find the forces in the string using Lame’s theorem and by equation of statics.

18.	a.	A smooth circular cylinder of radius 1.5m is lying in a triangular groove, one side of which 	makes 	15° angle and the other 40° angle with the horizontal. Find the reactions at the surfaces of 	contact, if there is no friction and the cylinder weighs 100N.							(8)

b.	Two equal heavy spheres of 50mm radius are in equilibrium within a smooth cup of 150mm radius. Show that the reaction between the cup of one sphere is double than that between the two spheres.                                                      																			(7)

(OR)

[P.T.O]

19.	a.	Explain the types of friction with examples.                                                 				(5)

b.	Two equal bodies A and B of weight ‘W’ each are placed on a rough inclined plane. The bodies are 	connected by a light string. If μA = 1/2 and μB = 1/3, show that the bodies will be both on the point 	of motion when the plane is inclined at tan−1 (5/12).                  							(10)

20.	a.	Derive an expression to determine the moment of inertia of a semi circle about its diametric base.

(7)

b.	Find the moment of inertia of the shaded area, as shown in figure about its centroidal axes	parallel to x-axis.                                                            								(8)

(OR)

21.	 a.	Determine the moment of inertia of a hollow circular section about its centroidal axes as shown 	in figure.                                                           										(7)

b.	Write short notes on the mass moment of inertia of thin circular and rectangular plates. 		(8)

22.	a.	A body moves along a straight line and its acceleration ‘a’ which varies with time ‘t’ is 	given by

a= 2-3t. Five seconds after the start of observation, the velocity is 20 m/s. The distance moved by 	the body 10sec after the start of observation of motion from origin is 85m.

Determine

- the acceleration, velocity and distance from the origin at the start of observation.

ii.	the time after the start of observation at which the velocity becomes zero and the distance 	travelled from the origin.                                                      			 				(7)

b.	A car is uniformly accelerated and passes successive kilometer stones with velocities of 	20km/hour and 30km/hour respectively. Calculate its velocity when it passes the next 	kilometer 	stone and the time taken for each of these two intervals of one kilometer.					(8)

(OR)

23.	A projectile is aimed at a mark on the horizontal plane through the point of projection and falls 12m short when the angle of projection is 150 while it overshoots the mark by 24m when the angle of projection is 450. Find the angle of projection to hit the mark neglecting the resistance of air.  Take g = 9.81 m/s2.

24.	a.	The motion of a particle in rectilinear motion is defined by the relation s = 2t3−9t2 +12t−10 	where 	s is expressed in metres and t in seconds. Find (i) the acceleration of the particle 	when the 	velocity is zero  (ii) the position and the total distance traveled when the 	acceleration is zero.(7)

b.	Find the acceleration of the moving loads as shown in figure. Take mass of P=120 kg and that of Q=80 Kg and coefficient of friction between surfaces of contact is 0.3. Also find the tension in the connecting string.                      													(8)

(OR)

25.	Write short notes on

a.	Kinetic and potencial energy				b.	Momentum and impulse connections

c.	Conservation of mechanical energy		d.	Oblique central impact.