Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	FLUID MECHANICS AND MACHINERY				       Time: 3 hours

Subject Code:	CE212		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define fluid with an example.

2.	Specific gravity of oil is 0.9, Find specific weight.

3.	Define pressure at a point.

4.	Continuity equation in differential form for a three dimensional flow is ______.

5.	Write Bernoulli’s Equation.

6.	Distinguish between Major loss and Minor loss.

7.	Define slip of a pump.

8.	Distinguish between a pump and a turbine.

9.	State Impulse momentum equation.

10.	What is an air vessel?

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between Surface Tension and Capillarity.

12.	The stream function in a two dimensional flow is given by Ψ=x2-y2, determine the velocity at point (1,1).

13.	List out the various types of losses in pipe flow.

14.	How are turbines classified?

15.	Write short notes on selection of a pump.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Distinguish between dynamic viscosity and kinematic viscosity and derive the units for 	viscosity.

b.	Calculate the density, specific weight and specific volume of 2 litres of petrol, its specific

gravity being 0.7.

(OR)

17.	a.	Derive an expression for force exerted by a curved surface submerged in a fluid.

b.	A simple U-tube manometer containing mercury is connected to a pipe in which a fluid of specific gravity 0.8 and having vacuum pressure is flowing. The other end of the manometer is open to atmosphere. Find the vacuum pressure in a pipe, if the difference of mercury level in the two limbs is 40 cm and the height of the fluid in the left from the centre of pipe is 15 cm below.

18.	a.	Derive three dimensional continuity equation for fluid flow.

b.	The velocity components in a two dimensional incompressible flow is V= Y3+ 6x - 3x2y, 	and U = 3y2x - 6y - x3. (i) Is the flow continuous, (ii) Is the flow Irrotational  (iii)) In case 	the 	flow is irrotational , find potential function and stream function.

(OR)

[P.T.O]

19.	a.	Water flows through a pipe AB is 1.2 m diameter at 3m/sec and passes through the pipe 	BC 1.5m diameter, At C, the pipe branches, Branch CD is 0.8 m in diameter and carries 	one third of flow in AB. The flow velocity in branch CE is 2.5 m/sec. Find (i) Volume rate 	of flow in AB, (ii) Velocity in BC, (iii) Velocity in CD and (iv) Diameter of CE.

b.	The velocity components for a two dimensional incompressible flow are U=2x + y and         	V= -x-2y, find stream function.

20.	a.	A 150 mm x 75 mm Venturimeter is placed vertically with throat diameter of 225 mm 	above the inlet conveys oil of specific gravity of 0.78 at 29 liters/sec. Calculate the 	difference in pressure between the inlet and the throat. Take Cd = 0.96.

b.	The difference in water surface levels in two tanks, which are connected by three pipes in series of length 300 m, 170 m, and 210 m and diameter 300 mm, 200 mm, and 400 mm respectively is 12 m. Determine the rate of flow if the coefficient of friction are 0.005,0.0058 and 0.0048 respectively, considering the minor losses and neglecting the minor losses.

(OR)

21.	a.	Derive Darcy’s Weisbach Equation.

b.	Water is flowing through a pipe of length100 m having a diameters 600 mm at the upper end and 300mm at lower end, at a rate of 50 litres/sec. The pipe as a slope of 1 in 30, Find the pressure at the lower end if the pressure at the higher level is 19.62 N/cm2.

22.	The diameter and the stroke of a single acting reciprocating pump are 200 mm and 400 mm respectively, the pump run at a60 rpm and lifts 12 litrres of water per second through a height of 25m. the delivery pipe is 20 m long and 150 mm diameter. Find i) theoretical power required to run the pump, ii) percentage of slip and acceleration head at the beginning  and middle of the delivery stroke.

(OR)

23.	a.	Draw a next sketch of a centrifugal pump and explain the operating principles of 	centrifugal pump

b.	A centrifugal pump is running at 1000 rpm, the vane angle of the impellor is 45° and the velocity of flow at the outlet is 2.5 m/sec. The discharge through the pump is 200 liters/sec when the pump is working under the total head of 20 m, If the manometric efficiency of the pump is 80 %, determine the diameter of the impellor and the width of the impellor at the outlet.

24.	An inward flow reaction turbine is supplied with 0.233m3/sec of water under a head of 11m.  The wheel vanes are radial at the inlet and the inlet diameter is twice the outer diameter.  The velocity of flow is constant and is equal to 1.83 m/sec. The wheel makes 370 rpm. Determine (a) Guide vane angle, (b) Inlet and outlet diameter of the wheel (c) Width of the wheel at inlet and the exit. Assume the discharge is radial and there is no loss in the wheel. Take speed ratio as 0.7. Neglect the thickness of the vanes.

(OR)

25.	A turbine is to operate under a head of 30 m at 250 rpm. The discharge is 10.5m3/sec, if the efficiency of the turbine is 85 %, determine (a) power generated, (b) specific speed of the turbine, (c) type of the turbine (d) performance under a head of 25 m.