Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MECHANICS OF DEFORMABLE BODIES - II			       Time: 3 hours

Subject Code:	CE241		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Write down the expression for the slope at the supports of a simply supported beam carrying a point load at center.

2.	Write down the expression by moment area method.

3.	Define column.

4.	What is equivalent length of a column?

5.	Differentiate between thin cylinder and thick cylinder.

6.	Which theory is best suited for ductile materials?

7.	Define shear centre.

8.	Write down the expression for Winkler-Bach formula.

9.	Define unsymmetrical bending.

10.	State the reason for unsymmetrical bending.

PART – B (5 x 3 = 15 MARKS)

11.	A cantilever of length 3 m is carrying a point load of 25 kN at the free end. If the moment of inertia of the beam is 1x108 mm4 and the value of E = 2.1x105N/mm2., find the deflection at the free end.

12.	Explain the assumptions made in the Euler’s column theory.

13.	Name the important theories of failure.

14.	What are the assumptions made in the derivation of stresses in a curved bar which is subjected to bending moment?

15.	State and explain the expression used for finding out the resultant bending stress at a point due to unsymmetrical bending.

PART – C (5 x 15 = 75 MARKS)

16.	A beam of length 6m is simply supported at its ends and carries two point loads of 48kN and 40kN at a distance of 1m and 3m respectively from the left support. Find deflection under each load and the maximum deflection.

(OR)

17.	A cantilever of length 2m carries a point load of 20kN at the free end and another load of 20 kN at its center. If E = 1x105N/mm2 and I = 1x108mm4 for the cantilever, determine, by moment area method, the slope and deflection of the cantilever at the free end.

18.	Calculate the Euler’s critical load for a strut of T-section, the flange width being 100mm, overall depth 80mm and both flange and stem being 10mm thick. The strut is 3m long and is built- in at both ends. Take E = 2x105 N/mm2.

(OR)

19.	A hollow cylindrical cast iron column is 4m long with both ends fixed. Determine the minimum diameter of the column if it has to carry a safe load of 250 kN with a factor of safety of 5. Take internal diameter as 0.8 times the external diameter. Take σc =550N/mm2 and                  α = 1/1600.

[P.T.O]

20.	Determine the maximum and minimum hoop stress across the section of a pipe of 400mm internal diameter and 100mm thick, when the pipe contains a fluid at a pressure of 8N/mm2. Also sketch the radial pressure distribution and hoop stress distribution across the section.

(OR)

21.	Determine the diameter of a bolt, which is subjected to an axial pull of 9kN together with a transverse shear force of 4.5kNm, using maximum principle stress theory and maximum principle strain theory. The value of elastic limit in tension is 225 N/mm2. Factor of safety is 3. Poisson’s ratio is 0.3.

22.	Derive an expression to locate the shear center for a channel section.

(OR)

23.	Determine the position of neutral axis and maximum and minimum stresses when a curved beam of circular section of diameter 100mm is subjected to pure bending moment of 11.5kNm. The radius of curvature is 100mm.

24.	A 80x80x10mm angle is used as a simply supported beam over a span of 2.4m. It carries a load of 400N along the vertical axis passing through the centroid of the section. Determine the resulting bending stresses on the outer corners of the section, along the middle of the section of the beam. Take E = 200GPa.

(OR)

25.	A beam of T-section (flange=100x20mm, web=150x10mm) is 2.5m in length and is simply supported at the ends. It carries a load of 3.2kN inclined at 20o to the vertical axis and passing through the centroid of the section. If E = 2 x105 N/mm2., calculate the maximum tensile stress and maximum compressive stress.