Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ENGINEERING MECHANICS							       Time: 3 hours

Subject Code:	CE242		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define Force.

2.	Define Resultant Force.

3.	What are the three equations of equilibrium?

4.	Define Coefficient of static friction.

5.	Define Centroid.

6.	Define Radius of gyration.

7.	Define Projectile.

8.	Define Coefficient of Restitution.

9.	Define Kinetic energy.

10.	State law of conservation of momentum.

PART – B (5 x 3 = 15 MARKS)

11.	Find the resultant of a 800N force acting towards eastern direction and a 500N force acting towards north eastern direction.

12.	State the laws of dry friction.

13.	A semi circular area having radius 100mm is located in the x-y plane such that its diametric edge coincides with y axis. Determine the x coordinate of its centroid.

14.	A body is moving with a velocity of 4m/s. After 5 seconds the velocity of body becomes 14m/s. Find the acceleration of the body.

15.	A mass of 15kg has an initial velocity of 15m/s horizontally on a smooth surface. Determine the value of the horizontal force that will bring the mass to rest in 4 seconds.

PART – C (5 x 15 = 75 MARKS)

16.	A string of length 310mm has its extremities attached to two fixed points situated 250mm apart in a horizontal line. If the string can bear any tension up to 36N, find the greatest load that can be supported at a point of the string distant 240mm from one extremity.

(OR)

17.	A cylindrical roller has a weight of 10kN and it is being pulled by a force which is inclined at 30o with horizontal. While moving it comes across an obstacle 10cm high. Calculate the force required to cross this obstacle, if the diameter of the roller is 1000mm.

16kN

12kN

60

45

30

30

10kN

4kN

A

B

C

D

18.	Four forces of magnitude and direction acting on a square ABCD of side 2m are shown in figure. Calculate the resultant in magnitude and direction and also locate its point of application with respect to side AB and AD.

(OR)

[P.T.O]

19.	A 7m long ladder rests against a wall with which it makes an angle of 45o on a floor. If a man whose weight is one half that of the ladder climbs it. At what distance along the ladder will he be, when the ladder is about to slip. Take coefficient of friction between the ladder and the wall is 1/3 and that between the ladder and the floor is ½.

20.	Find the moment of inertia of a T section of flange 100x30mm and web 20x80mm about its centroidal axes.

(OR)

4kN

60

30

A

B

C

2m

21.	Determine the magnitude and nature of forces in all the members of the truss shown in figure.

22.	Two trains A and B leave the same station on parallel lines. A starts with a uniform acceleration of 1/6 m/s2 and attains a speed of 24km/hr when steam is reduced to keep the speed constant. B leaves 40 seconds after with uniform acceleration of 1/3 m/s2 to attain a maximum speed of 48km/hr. When will it overtake A?

(OR)

23.	A particle is projected with an initial velocity of 60m/s at an angle of 75o with the horizontal. Determine the maximum height attained by the particle, horizontal range of the particle and time taken by the particle to reach highest point.

24.	A car is of mass 4500 kg .The tractive resistance on the level bank is 50N/tonne. Find the power required to propel the car at a uniform speed of 45km/hr. up an inclination of 1in 300 and on the level track. Assume the efficiency as 70%.

(OR)

25.	A bullet of mass 30 gm is fired horizontally into a body of mass 10kg, which is suspended by a string of 0.8m long. Due to this impact the body swings through an angle of 30o. Find the velocity of the bullet.