Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MECHANICS OF SOILS								       Time: 3 hours

Subject Code:	CE247		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define the term dry density.

2.	What is meant by sensitivity of clay?

3.	State Darcy’s law.

4.	What is meant by flow net?

5.	Define OMC.

6.	Define over consolidation ratio.

7.	Give the expression for total stress.

8.	What is meant by isobar?

9.	The strength envelope of clay is _______.

10.	Define stability number.

PART – B (5 x 3 = 15 MARKS)

11.	Write a note on phase diagram.

12.	Mention the applications of flow net.

13.	Distinguish between compaction and consolidation.

14.	Mention the uses of Newmark’s influence chart.

15.	List out the types of slope failure.

PART – C (5 x 15 = 75 MARKS)

16.	A soil sample has a porosity of 40%. The specific gravity of solids is 2.70. Calculate (a) Void ratio (b) dry density (c) unit weight if the soil is 50% saturated (d) unit weight if the soil is fully saturated and (e) water content.

(OR)

17.	Write a detailed note on Atterberg limit.

18.	In a falling head permeameter test, the initial head (t=0) is 40 cm. The head drops by 5 cm in 10 minutes. Calculate the time required to run the test for the final head to be 20 cm. If the sample is 6 cm in height and 50 sq.cm in cross sectional area, calculate the coefficient of permeability, taking area of stand pipe as 0.5 sq.cm.

(OR)

19.	Explain the factors on which permeability depends upon.

20.	Explain the consolidation process by spring analogy method.

(OR)

21.	Explain the field compaction method in detail.

[P.T.O]

22.	Find the intensity of vertical pressure and horizontal shear stress at a point 4 m directly below a 20 kN point load acting at a horizontal ground surface. What will be the vertical pressure and shear stress acting at a point 2 m horizontally away from the axis of loading but at the same depth of 5m?

(OR)

23.	A clay layer whose total settlement under a given loading is expected to be 12 cm settles 3 cm at the end of 1 month after the application of load increment. How many months will be required to reach a settlement of 6 cm? How much settlement will occur in 10 months? Assume the layer to have double drainage.

24.	Undrained triaxial tests are carried out on four identical specimens of silty clay and the following results are obtained:

Cell pressure (kN/m2) 	: 	50		100		150		200

Deviator Stress (kN/m2)	:	350		440		530		610

Pore Pressure (kN/m2)		:	5		10		12		18

Determine the value of the effective angles of shearing resistance and the cohesion intercept by plotting.

(OR)

25.	Explain friction circle method in detail.