Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	REINFORCED CONCRETE STRUCTURES - I			       Time: 3 hours

Subject Code:	CE249		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define neutral axis.

2.	What is a balanced failure?

3.	What is meant by equivalent shear?

4.	What are the functions of distributor bars in slabs?

5.	Define SBC of soil.

6.	Under what circumstances combined footings are resorted to.

7.	Write the equation for effective flange width in T and L- beams.

8.	Write the equation for moment in fixed circular slabs subjected to UDL.

9.	Define two way slab.

10.	What is an eccentrically loaded columns?

PART – B (5 x 3 = 15 MARKS)

11.	Comment on idealized stress – strain diagram of concrete and steel.

12.	What are anchorage stresses?

13.	What is an interaction curve?

14.	What are stirrups? Mention their role in beams subjected to Pure Bending and combined bending and shear.

15.	Write the working stress design concept.

PART – C (5 x 15 = 75 MARKS)

16. 	a.	Why limit state method is considered more rational than working stress method?		(5)

b.	Define the following terms.													(10)

i)	Partial safety factors and design values

ii)	Limit state of collapse and limit state of serviceability

(OR)

17.	Design a rectangular beam 400mm wide which is subjected to the following load.

Bending moment    = 45 kNm

Shear force             = 30 kN

Torsional moment  = 25 kNm

18.	Design a two – way slab for a room having clear dimensions of 3m x 5m. Take Live load intensity as 2 kN/sq.m and Load due to finishes as 1.6 N/sq.m.

(OR)

19.	a.	What is theoretical bar cut-off point? Why does the code disallow curtailment of flexural 	tension reinforcement at this point?												(4)

b.	Design a one – way slab with a clear span 4m simply supported on 230mm thick masonry wall and subjected to a live load of 3 kN/sqm and a surface finish load of 1 kN/sqm using Fe415 steel. Assume that the slab is subjected to very severe exposure. 				(11)

[P.T.O]

20.	Design a square column of 400mm side to carry factored axial load of 2000kN and factored bending moment of 150kNm.  The reinforcement is placed on (i) the opposite faces and (ii) all four faces.  Adopt LSD.

(OR)

21.	A solid footing has to transfer a dead load of 1200 kN and a imposed load of 550 kN from a square column 400 x 400mm.  Use M20 grade concrete and Fe415 steel. SBC of the soil at site is 200kN/sq.m. Design the footing.

22.	Design a singly reinforced concrete beam of clear span 4.5m with a bearing at each end as 200mm.  The beam is subjected to a UDL of 12 kN/m over its entire span and a central concrentral load of 30 kN.  Design the beam for flexure and check for deflection. Use working stress concept.

(OR)

23.	a.	Inclined stirrups and bent-up bars are considered suitable for shear reinforcement? Why?

(4)

b.	If the beam in Problem No. 22 is provided with 8mm plain bar at a uniform spacing of 220mm c/c, check the adequacy of the shear design.  If necessary, revise the design.	(11)

24.	Design Problem No. 18 using working stress concept.

(OR)

25.	Design steel required for a column of size 400 mm x 600mm and of unsupported length 3m subjected to an axial load of 2500 kN.  Use M20 grade concrete and Fe415 steel.