Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	STRUCTURAL ANALYSIS - I							       Time: 3 hours

Subject Code:	CE254		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the degree of freedom for fixed support?

2.	What is the use of Castigliano’s first theorem?

3.	What is EUDL?

4.	An udl of length shorter than the span crosses a girder from left to right. What should be the position of the load to cause maximum reaction at the right hand support?

5.	What is an arch? Explain.

6.	Under what conditions will the bending moment in an arch be zero throughout?

7.	Write down the expression to find the length of a suspension cable.

8.	What is a Catenary?

9.	Define Lack of fit.

10.	What is the other name of Clapeyron’s theorem?

PART – B (5 x 3 = 15 MARKS)

11.	State Betti-Maxwell’s law.

12.	Write the equation of theorem of three moments for a two span continuous beam.

13.	What are the uses of Influence Line diagrams?

14.	Which of the two arches, viz. circular and parabolic is preferable to carry an UDL? Why?

15.	Give atleast 3 differences between a plane truss and a space truss.

PART – C (5 x 15 = 75 MARKS)

4m

4m

8m

A

B

C

20kN/m

16. 	Determine the vertical deflection at free end C of the beam ABC shown in figure. Consider E = 2 x 105 N/mm2 and I = 8 x 108 mm4.

(OR)

17.	Determine the reaction VB for the continuous beam shown in figure by the method of least work.

8m

X

5m

2m

2m

2m

7m

20m

80kN

80kN

80kN

80kN

A

18.	Four equal loads of 80 kN each, equally spaced at 2 meters apart followed by a uniformly distributed load of 60 kN/m run at a distance of 2m from the last 80 kN load cross a girder of 20 meters span from right to left as shown in figure. Using influence lines, calculate the SF and BM at a section 8 meters from the left hand support, when the leading 80 kN is 5 meters from this support.

60kN/m

B

(OR)

[P.T.O]

19.	A uniform load of 25 kN/m, 6 meters long, crosses a girder of 30 meters span. Calculate the maximum SF and BM at sections 5, 10 and 15 meters from the left hand support. Construct the maximum SF and BM diagrams.

20.	A three hinged semicircular arch of span 16m and rise 4m is subjected to two point loads of 100kN and 80kN at the left and right quarter span points respectively. Find the reactions at supports. Find also the bending moment, radial shear and normal thrust at 6m from left support.

(OR)

21.	A two hinged parabolic arch has a varying moment of inertia given by I=Iosecθ. It has a span of 40m and a central rise of 8m. Calculate the maximum positive and negative bending moments at a section D, 12m from the left support, due to a moving point load of 6kN.

22.	Using the tension coefficient method, calculate the forces in the members of the pin-jointed space frame shown in figure. The numbers in parentheses are the Cartesian coordinates of the joints of the frame.

(OR)

23.	A suspension cable 140 m span and 14 m central dip carries a load of 1 kN/m. Calculate the maximum and minimum tension in the cable. Find the horizontal and vertical forces in each pier under the following conditions:

a.	If the cable passes over a frictionless rollers on the top of the piers

b.	If the cable is firmly clamped to saddles carried on frictionless rollers on top of piers.

c.	In each case, the backstay is inclined at 30o with the horizontal.

24.	A frame work of six bars of uniform sectional area and hinged together to form a square of 750mm sides with two diagonals is suspended from one end as shown in figure. At the opposite corner, a load of 50kN is suspended. Calculate the force in each bar using Consistent Deformation method.

(OR)

25.	Determine the support moments and construct shear force and bending moment diagrams for the beam shown in figure. Take EI constant. Adopt Clapeyron’s theorem.

20kN/m

A

B

C

6m

4m