Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title : 	MECHANICS OF FLUIDS								      Time : 3 hours

Subject Code:	CE277			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Write the relationship between mass density and weight density.

2.	Name the simplest form of manometer used to measure the moderate pressure of liquids.

3.	Write the continuity equation for two dimensional flows.

4.	What is the path line?

5.	Name the equipments used to measure the discharge from a tank.

6.	List the different types of notches.

7.	Name the various types of minor losses.

8.	What is the condition for the maximum power transmission through pipes?

9.	Models which are not geometrically similar are known as _________.

10.	Reynolds number is the ratio of _________ force and __________ force.

PART – B  (5 x 3 = 15 MARKS)

11.	State Newton’s law of viscosity.

12.	Define ‘Velocity Potential function’.

13.	Differentiate Notches and weirs.

14.	What do you mean by pipes in parallel?

15.	What are the problems for which Reynolds model law is applicable?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Define ‘Buoyancy and Meta-centric Height’.										(5)

b.	A rectangular plate 3m long and 1m wide is immersed vertically in water in such that its 3m side is parallel to the water surface and is 1m below it. Find: (i) Total pressure on the plate and (ii) Position of centre of pressure.										(10)

(OR)

17.	a.	Derive an expression for the capillary rise.										(5)

b.	A vertical cylinder of diameter 180mm rotates concentrically inside another cylinder of diameter 181.2mm. Both the cylinders are 300mm high. The space between the cylinders is filled with a liquid whose viscosity is unknown. Determine the viscosity of the fluid if a torque of 20Nm is required to rotate the inner cylinder at 120rpm.					(10)

18.	a.	Derive the continuity equation for three dimensional flow.							(8)

b.	The Velocity components in a two dimensional incompressible flow are, u=y3+6x-3x2y,

v=3yx2-6y-x3; (i) Is the flow continuous?, (ii) Is the flow irrational?, (iii) In case the flow is Irrotational find the potential function and the stream function.  					(7)

(OR)

19.	a.	Derive the Bernoulli’s equation for a streamline flow from the Euler’s equation.		(9)

b.	A 400m long pipe tapers from 1.20m diameter at the high end to 0.60m diameter at the low end, the slope of the pipe being 1 in 100. The pipe conveys a discharge of 1.25m3/s. If the pressure at the high end is 75kN/m2, find the pressure at the low end. Ignore losses.	(6)

[P.T.O]

20.	a.	Find the discharge from a 80mm diameter external mouthpiece, fitted to a side of a large

vessel, if the head over the mouthpiece is 6m. Assume the coefficient discharge as 0.855.								    	   												(6)

b.	A horizontal venturimeter with inlet diameter 200mm and throat diameter 100mm is

employed to measure the flow of water. The reading of the differential manometer connected to the inlet is 180mm of mercury. If the coefficient of discharge is 0.98, determine the rate of flow.							   							(9)

(OR)

21.	a.	A rectangular notch has a discharge of 0.24m3/s, when head of water is 0.8m.Find the

length of the notch. Assume coefficient of discharge as 0.6.	    	  				(6)

b.	Oil of specific gravity 0.88 is flowing through a pipe of diameter 240mm and an orifice

meter is used for measuring the discharge. The diameter of the orifice is 120mm. The reading of the differential manometer connected to the inlet is 400mm of mercury. If the coefficient of discharge is 0.65, determine the rate of flow of the oil.				(9)

22.	Derive the Hagen – Poiseullie equation for the laminar flow through a circular pipe.

(OR)

23.	a.	A piping system consists of three pipes arranged in series; the lengths of the pipes are

1200m, 750m and 600m and diameters 750mm, 600mm and 450mm respectively.

i.	Transform the system to an equivalent 450mm diameter pipe and

ii.	determine an equivalent diameter for the pipe, 2250m long.			    		(6)

b.	A horizontal pipeline 40m long is connected to a water tank at one end and discharges

freely into the atmosphere at the other end. For the first 25m of its from the tank, the pipe is 150mm diameter and its diameter is suddenly enlarged to 300mm. the height of water level in the tank is 8m above the centre of the pipe. Considering all losses of head which occur. Determine the rate of flow. Take f=0.01 for both sections of the pipes.		(9)

24.	The frictional torque T of a disc of diameter D rotating at a speed N in a fluid of viscosity µ and density ρ in a turbulent flow is given by T=D5N2ρφμD2Nρ. Prove by Buckingham’s Pi method.

(OR)

25.	a.	What are the advantages of model analysis. 										(5)

b.	A ship 300m long moves in sea-water, whose density is 1030kg/m3. A 1:100 model of this ship is to be tested in a wind tunnel. The velocity of air in the wind tunnel around the model is 30m/s and the resistance of the model is 60N. Determine the velocity of ship in sea water and also the resistance of the ship in sea water. The density of air is given as 1.24kg/m3. Take the kinematic viscosity of sea water and air as 0.012 stokes and 0.018 stokes respectively.															(10)