Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	SURVEYING											       Time: 3 hours

Subject Code:	CE280		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Classify surveying.

2.	Name the various types of levels commonly used.

3.	Name the various methods for measuring horizontal angle.

4.	What is meant by transiting of theodolite?

5.	State the advantage of using anallatic lens.

6.	State the use of alidade.

7.	State the relationship between radius and degree of curve.

8.	What is meant by vertical curve?

9.	Classify triangulation system.

10.	What is meant by reduction to centre?

PART – B (5 x 3 = 15 MARKS)

11.	Compare the height of instrument and rise and fall method used for calculation of reduced level.

12.	What are the temporary adjustments to be done for a theodolite.

13.	State the advantages of plane table survey.

14.	State the functions of transition curve.

15.	A steel tape 20m long was standardized at 25C. Find the correction per tape length if the temperature at the time of measurement was 40C. Coefficient of thermal expansion             =11 x 10-6 per C.

PART – C (5 x 15 = 75 MARKS)

16.	Name the different methods of contouring. Explain any two methods in detail with sketches.

(OR)

17.	The following records refer to an operation involving reciprocal leveling

| Instrument at | Staff Reading on |  | Remarks |
| --- | --- | --- | --- |
|  | A | B |  |
| A | 1.155m | 2.595m | Distance AB = 500m, |
| B | 0.985m | 2.415m | RL of A = 525.5m |

Find the true R.L. of B, and the combined correction for curvature and refraction.

18.	Explain in detail any two methods by which closing error in a traverse can be balanced.

(OR)

19.	In order to ascertain the elevation of the top (Q) of the signal on a hill, observations were made from two instrument stations P and R at a horizontal distance 1000m apart, the stations P and R being in line with Q. The angles of elevation of Q at P and R were 2842 and 186 respectively. The staff readings upon the BM of elevation 287.28 m were respectively 2.87 and 3.75 m when the instrument was at P and R, the telescope being horizontal. Determine the elevation of the foot of the signal if the height of the signal above its base is 3m.

[P.T.O]

20.	A tacheometer was set up at a station A and the following readings were obtained as a staff held vertically. Instrument is fitted with anallatic lens R.L. of B.M. is 100 m.

| Instrument station | Staff station | Vertical angle | Hair readings |
| --- | --- | --- | --- |
| A | B.M | -5 20 | 1.15, 1.8, 2.45m |
| A | B | 812 | 0.75, 1.5, 2.25m |

Calculate horizontal distance AB and R.L. of B.

(OR)

21.	Explain how you will solve two-point problem.

22.	 Explain the various surveys to be conducted for the location of a railway project.

(OR)

23.	Name the various methods for setting out a simple curve. Explain any two methods for setting out simple curve.

24.	The altitude of two proposed stations A and B 130 km apart are respectively 220m and 1160m. The altitudes of two points C and D on the profiles between them are respectively 308m and 632 m , the  distance AC = 50 km and AD = 90km . Determine whether A and B are intervisible, and if necessary, find the minimum height of a scaffolding at B assuming A as ground station.

(OR)

25.	Write a detailed notes on the following:												(9+6)

a.	Signals and towers			b.	Selection of triangulation station