Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	CHEMICAL REACTION ENGINEERING				       Time: 3 hours

Subject Code:	CL205		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are elementary reactions?

2.	Name the types of Ideal Reactors.

3.	Define Space Time.

4.	Classify multiple reactors.

5.	State the reasons for Non-ideality in reactors.

6.	Name the experimental methods for determination of residence time distribution.

7.	Define Thiele modulus.

8.	Write the rate equation of gas liquid instantaneous reactions with respect to mass transfer.

9.	Mention the factors to be considered for selecting contactor for fluid- fluid operation.

10.	Write the design equation for plug flow reactor.

PART – B (5 x 3 = 15 MARKS)

11.	Derive the rate equation for second order reaction 2A  Products.

12.	Write the performance equation for equal size mixed flow reactor for first order reaction.

13.	Mention the factors that affect conversion in non-ideal reactors.

14.	Discuss the significance of Hatta number in fluid fluid reactions.

15.	Write the applications of Slurry reactors.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Discuss about the temperature dependency of the rate of the reaction. 				(7)

b.	Milk is pasteurized if heated to 63°C for 30 min but if it is heated to 74°C it needs only 15s 	for the same result. Find the activation energy of the sterilization process.		     	(8)

(OR)

17.	Reactant A decomposes in a batch reactor A  R. Find the rate expression using differential method of analysis

| Time S | 0 | 20 | 40 | 60 | 120 | 180 | 300 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA mol/l | 10 | 8 | 6 | 5 | 3 | 2 | 1 |

18.	a.	Derive the performance equation for Plug flow reactor.   							(7)

b.	A homogenous gas phase reaction A   3R has a reported rate at 215°C -rA = 0.01 CA1/2 	mol/lit s. Find the space time for 80% conversion of 50 % A and 50 %  inert feed  to 	a                 	Plug flow reactor with CAo = 0.0625mol/lit. 										(8)

(OR)

19.	Discuss the performance of mixed flow reactor of different size in series to find the conversion in a given system and to determine the best system for given conversion.

20.	Derive the expression for tank in series model for non-ideality in reactors.

(OR)

21.	Explain in detail about dispersion model for a chemical reaction of first order.

[P.T.O]

22.	Derive the rate expression for surface reaction controlling fluid solid catalytic reaction.

(OR)

23.	Derive the expression for effectiveness factor for pore diffusion resistance combined with surface kinetics.

24.	Explain with neat sketch the construction and working principle of Fluidized bed reactor.

(OR)

25.	Explain briefly about the working principle of slurry reactors and trickle bed reactor.