Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	UNIT OPERATIONS									       Time: 3 hours

Subject Code:	CL222		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The overall resistance for heat transfer through a series of flat plat is the _______ if the resistance.

2.	Unsteady state conduction occurs when _______.

3.	Prandtl number is the reciprocal of _______.

4.	The absorptivity of a black body is _______.

5.	The purpose of floating head in a heat exchanger is to _______.

6.	In a backward feed multiple effect evaporator feed flows from _______pressure to _______ pressure.

7.	Filter aid is used to increase _______.

8.	The specific cake resistance for incompressible sludge is independent of _______.

9.	A propeller agitator produces mainly _______ flow.

10.	Helical screw agitator is used for mixing _______ pastes.

PART – B (5 x 3 = 15 MARKS)

11.	Define heat flux and convective heat transfer coefficient.

12.	Give the significance of Prandtl number.

13.	How will you measure the performance of an evaporator?

14.	Write the advantages of cross flow filtration over conventional filtration.

15.	What are the purposes of agitation?

PART – C (5 x 15 = 75 MARKS)

16.	a.	A thick walled tube of stainless steel having thermal conductivity 21.63 W /m K with 	dimension of 0.0254m inner diameter and 0.0508m outer diameter is covered with 	0.0254m layer of asbestos insulation of thermal conductivity 0.2423 W /m K. The inside 	wall temperature of the pipe is 811 K and the outer surface of the insulation is at 310 K for 	a 0.305m length pipe. Calculate the heat loss and also the temperature at the interface 	between the metal and the insulation.							  	  			(10)

b.	Derive an expression for determining the rate of heat transfer for conduction through a 	thick walled hollow sphere.	   						   	   					(5)

(OR)

17.	a.	Derive the expression for overall heat transfer coefficient from individual heat transfer 	coefficient based on outside area.      		 					  				(10)

b.	Derive an expression for combined conduction, convection and overall heat transfer 	coefficient for a flat wall.				   				   						(5)

18.	a.	Discuss the heat transfer coefficient mechanisms for laminar and turbulent flow inside a 	pipe. 											   							(5)

b.	By dimensional analysis, show that Nusselt number is a function of Reynolds number and Prandtl number for a fluid flowing in turbulent flow inside a pipeline.	  	  		(10)

(OR)

[P.T.O]

19.	a.	Saturated steam at 267 F is flowing inside a ¾ inch steel pipe having an ID of 0.824m and 	OD of 1.050 inch. The pipe is insulated with 1.5 inch if insulation on the outside. The 	convective coefficient for the inside steam surface of the pipe is estimated as 1000 btu / hr 	ft2 F and that of the outside is 2 btu / hr ft2 F. The mean thermal conductivity of the metal 	is 26 btu / hr ft F and 0.037 btu / hr ft F for insulation. Calculate the heat loss for 1 ft of 	pipe using resistance if the surrounding air is at 80F.				              	  		(10)

b.	Explain the principle of drop wise and film type condensation.		   				(5)

20.	a.	Derive mass and enthalpy balance for single and multiple effect evaporators.		(10)

b.	What are the liquid characteristics, which affects the design and operation of evaporators? 	Explain.	   								   									(5)

(OR)

21.	a.	Derive an expression for LMTD based on the parallel flow heat exchanger.	    		(7)

b.	Find the heating area required for concentrating a solution of 5000 Kg/hr of 10% NaOH 	entering at 30°C to a concentration of 50% NaOH evaporation is carried out in a single 	effect and the overall heat transfer coefficient is 2500 W/m² K. The evaporator is operated 	at 245 mmHg. Find the economy of the evaporator. Boiling pt of water at 254 mmHg is 	70°C, Boiling pt of 50% NaOH is 110°C, Steam temperature is 150°C, Latent heat of 	steam is 510 kJ/kg, Enthalpy of feed is 35 kJ/kg, Enthalpy of product is 35 kJ/kg and 	enthalpy of vapor leaving is 600 kJ/ kg. 										    	(8)

22.	Explain with neat diagram the different types of filtration used in industries and their applications.

(OR)

23.	a.	Using the filter press with an area of 0.0929 m2 performed constant-pressure filtration at 	35.5 kPa of 13.9 wt % CaCO3 solid in water slurry at 300K. The mass ratio of wet cake to 	dry cakes was 1.59. The dry cake density was 1017 kg/m3. The data obtained are as 	follows, where W is the kg of filtrate and t is the time in second. Determine the filter 	medium resistance and specific cake resistance.							    	  	(10)

| Weight ( kg) | 0.91 | 1.81 | 2.72 | 3.63 | 4.54 | 5.44 | 6.35 | 7.26 | 8.16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time (sec) | 24 | 71 | 146 | 244 | 372 | 524 | 690 | 888 | 1188 |

b.	Derive the expression for constant - rate filtration to obtain filter medium resistance and 	specific cake resistance.					    			  						(5)

24.	a.	A disk turbine with 6 blades is installed centrally in a vertically baffled tank 1.5 mts in dia. 	The turbine is 0.65 mt in dia and is positioned 0.65 mt above the bottom of the tank. The 	turbine blades are 134 mm wide. The turbine impeller turns at 90 rpm. The tank is filled 	with a rubber latex compound having a viscosity of 120 pa.sec and density of 1120 Kg/m3. 	What power in kW will be required to drive the impeller? (Take KL = 65)		    	(9)

b.	What are the different method used for preventing swirling and vortex formation in an 	agitation vessel with neat diagram? Explain.					                			(6)

(OR)

25.	a.	Explain the equipments used for mixing of pastes and powder.		    				(9)

b.	Derive an expression for power consumption in an agitation system for Non –newtonian 	fluids.          										    							(6)