Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTRONICS AND MICROPROCESSORS			       Time: 3 hours

Subject Code:	EC213		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	How are collector-base and base-emitter junctions of BJT biased in the active region?

2.	What type of feedback is used in oscillators?

3.	What is the expected voltage range of Transducers?

4.	Mention the uses of potentiometer.

5.	Mention the characteristics table of D flip-flop.

6.	How many flip-flops are required for designing a Mod-10 counter?

7.	How many bits are supported by 8085 processor data bus and address bus?

8.	In 8085 processor, ALE signal falls under which bus?

9.	What type of operation is supported by RAM chips?

10.	Which data transfer scheme is the fastest one?

PART – B (5 x 3 = 15 MARKS)

11.	State Barkhausen criterion.

12.	What is piezoelectric effect?

13.	Convert the given Hex number (B65F)16  into an equivalent binary and decimal numbers.

14.	Write an ALP for finding the two’s complement of a given binary number using 8085.

15.	Differentiate memory mapped I/O and I/O mapped I/O.

PART – C (5 x 15 = 75 MARKS)

16.	In detail, explain the operation of the full wave rectifier circuit with relevant input and output waveforms.

(OR)

17.	Draw the circuit of a phase shift oscillator. Explain its operation and obtain its frequency of oscillation expression.

18.	Explain the working principles of strain gauges with relevant diagrams in detail.

(OR)

19.	Write short notes on the operations of following devices:

a.	Photo Transistors 	b.	Thermistors         										(8+7)

20.	a.	Explain the operation of a multiplexer circuit with its function table.   				(10)

b.	Explain the operation of JK flip-flop with its function table.                				(5)

(OR)

21.	Explain the operation of a 4-bit ripple counter with its timing diagram.

22.	Explain the architecture of Intel 8085 microprocessor in detail.

(OR)

23.	Explain the arithmetic and logical instructions of 8085 Microprocessor with an example.

[P.T.O]

24.	Draw the interfacing logic of memory devices and explain the same in detail. Assume appropriate size of the ROM and RAM.

(OR)

25.	Explain the different types of data transfer schemes in detail. Also mention the important features of each.