Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTROMAGNETIC FIELDS							       Time: 3 hours

Subject Code:	EC233		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	State the condition for the vector F to be solenoidal.

2.	A pitch ball is suspended by a non-conducting fiber. When a positively charged rod is brought near the ball, but without touching it, the ball:

a.	becomes positively charged 		b.	becomes negatively charged

c.	becomes depolarized 				d.	becomes polarized

3.	Which of the statement is not a characteristic of static magnetic field?

a.	it is solenoidal 	b.	it is conservative 		c.	it has no sink/source

d.	magnetic flux lines are closed

4.	Whether Ampere circuital law can be used to determine H when the current distribution is asymmetrical?

5.	Define current density.

6.	Define dielectric strength.

7.	Ferromagnetic material lose their nonlinearity property above the curie temperature.

(True / False)

8.	Define Magnetization.

9.	The Poynting vector physically denotes the power density leaving or entering a given volume in a time-varying field.  (True / False)

10.	A loop is rotating about the y-axis in a magnetic field B = Bo sin ωt ax wb/m2. The voltage induced in the loop is due to (a) transformer emf (b) a combination of both (c) motional emf (d) none of the above.

PART – B (5 x 3 = 15 MARKS)

11.	Find the gradient of r=x2+y2+z2.

12.	Magnetic vector potential A = -ρ2/4az wb/m. Calculate total magnetic flux crossing the surface φ = π/2, 1≤ ρ ≤ 2, 0 ≤ z ≤ 5.

13.	A parallel plate capacitor consists of two plates each of 30 cm x 30 cm, separated 2 mm apart and dielectrics each 1 mm thick have εr of 3 and 5. If potential difference between the plates is 5 kV, calculate voltage in each dielectric.

14.	The toroidal core has ρ0 = 10 cm and a circular cross section with a = 1cm. If the core is made of steel (μ = 1000 μ0) and has a coil with 200 turns. Calculate the amount of current that will produce a flux of 0.5 mWb in the core.

15.	Distinguish between field and circuit theory.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Obtain the flux of D = (5 r2 / 4) ar over the closed volume of sphere r = 1 and r = 2 .Also 	verify divergence theorem. 													(10)

b.	Transform the vector A = 4ax - 2ay - 4az at point (2, 3, 4) into cylindrical coordinate 	system.																		(5)

(OR)

[P.T.O]

17.	a.	A spherical volume charge density distribution is given by, ρ=ρ01−r2a2; r≤a.

Calculate the total charge Q and find the electric charge E inside sphere.			(5+5)

b.	The Two point charges Q = 2 nc and Q = -4nc are located at (1, 0, 3) and   (-2, 1, 5) 	respectively. Determine the potential at P (1,-2, 3).								(5)

18.	a.	A small current loop L1 with magnetic moment 5az A-m2 is located at the origin while 	another small loop current L2 with magnetic moment 3ay A-m2 is located at (4, -3, 10). 	Determine the torque on L2.													(10)

b.	In a region where µ = 4.6 µ0 has B = 10 e-y az mWb/m2. Find (i) Magnetic Susceptibility, χm (ii) Magnetic field intensity (iii) Magnetization.								(5)

(OR)

19.	Obtain the Magnetic field intensity everywhere of an infinite long coaxial cable having two concentric cylinders kept along z-axis. The inner conductor has radius ‘a’ and carries ‘I’ amps while the outer conductor has inner radius ‘b’ and outer radius ‘c’ and carries ‘–I’ amps.

20.	A capacitor consists of two squared metal plates each having 100 cm side is parallel and placed 2 mm apart. The space between the plates is filled with a dielectric having a relative permittivity of 3.5. A potential drop of 500 V is maintained between the plates. Calculate the capacitance, the charge of capacitor, the electric flux density, the potential gradient and the energy stored.

(OR)

21.	Two conducting spherical shells with radii a = 10 cm and b = 30 cm are maintained at a potential difference of 100 V such that V(r = b) = 0 and V(r = a) = 100 V. Determine the potential V and field intensity E in the region between the shells.

22.	The interface 2x + y = 8 between two magnetic media carries no current.  If medium-1          (2x + y≥ 8) is nonmagnetic with H1 = -4ax + 3ay – az A/m. Find (i) magnetic energy density in medium-1,(ii) M2 and B2 in medium-2 (2x + y ≤ 8) with µ = 10 µ0, (iii) the angles H1 and H2 make with the interface.

(OR)

23.	Derive the potential and electric field intensity of a dipole having charge ‘q’ separated by a distance of ‘l’ m and which is at a distance of ‘r’ m from the centre point to the measuring point.

24.	a.	Derive all Maxwell’s equation in Point form and Integral form.					(10)

b.	A straight conductor of length 40 cm moves perpendicular to its axis at a velocity of 50 m/s in a uniform magnetic field of B = 1.2 Tesla. Evaluate emf induced in conductor if direction of motion is (i) normal to field (ii) parallel to field (iii) at an angle 600 to the orientation of field.			 												(5)

(OR)

25.	Determine the self-inductance of a co-axial cable of inner radius ‘a’ and outer radius ‘b’ which carries current of ‘I’ amps and ‘–I’ amps respectively.